Signature Unit Safe Driver

User Guide

Project PdM DI [Signature Unit Driver] (Design)

Dinakar Babu Rodda

September 2, 2020, 03:25 PM IST

- Table of Contents

1	Revision History	4

2	Introduction	5

2.1	Purpose	5

2.2	Scope	5

2.3	Acronyms	5

3	Overview	5

3.1	Signature unit	5

3.1.1	Signature computation	5

3.1.2	Reference Check	6

3.1.3	Evaluation windows	6

3.1.4	Alpha masking	6

3.1.5	Panic modes	6

3.2	Safety Features	7

3.3	Software Functions	7

3.4	Operating Environment	7

3.5	Design and Implementation Constraints	7

3.6	Assumptions and Dependencies	7

4	External Interface	8

4.1	Hardware Interfaces	8

4.2	Software Interfaces	8

4.3	Debug Interfaces	8

5	Functionality	8

5.1	Resource Manager for signature unit	8

5.1.1	Initialization	8

5.1.2	Evaluation control and Configuration	9

5.1.3	Window status and CRC	9

5.1.4	Shutdown	10

5.2	API Library	10

6	Signature Unit BSPAL User Guide	10

6.1	BSPAL API Usage	10

6.2	Copy the binaries and libraries	12

6.3	Errors and Fixes	12

6.4	Run Application	13

7	Non Functional Requirements	13

7.1	Functional Safety Requirements	13

7.2	Security Requirements	13

7.3	Software Quality Attribute	14

7.3.1	Maintainability	14

7.3.2	Portability	14

7.3.3	Reusability	14

7.3.4	Testability	14

8	Reference	14

# Revision History

| Version | Date | Author | Description of changes |
| --- | --- | --- | --- |
| 0.1 | 2 Sept 2020 | Dinakar Babu Rodda | With user guide |

# Introduction

In general, to control the correctness of the display output. The signature values of the image can be computed for each frame and compared against reference values. In case of mismatch in signature values (violation) a HW event can be triggered

## Purpose

The purpose of this document is to describe the design for ASIL-B on Signature unit Safe driver and its interfaces.

## Scope

The Signature Unit Safe driver is used to configure the Signature unit and values of the image can be computed for each frame and compared against reference values.

## Acronyms

| Acronyms | Description |
| --- | --- |
| ASIL | Automatic Safety Integrity Level |
| DCIC | Display Content Integrity Checker |

# Overview

Signature unit and safety feature are the major blocks on this requirements to verify the Check sum of the image.

## Signature unit

In order to control the correctness of display output, signature values can be computed for each frame and compared against reference values. In case of a mismatch (signature violation) a HW event can be triggered, for example a SW interrupt. Below brief feature on Signature unit,

- 2 Signature Unit in IMX8.

- Signature unit (CRC checksum; up to 8 independent windows) for each display

- Windows checked in parallel for each frame

- Individual windows can be exempt from check

- Use of alpha mask for evaluating of arbitrary window shapes

- Frame Error Counter with programmable Threshold

- Automatic reaction on mismatch if expected content is not shown

- Generation of SW interrupt

- Automatic HW switch to constant color mode for individual evaluation windows

- Operation modes: Continuous (each frame) and single frame triggered by SW.

### Signature computation

- Start value is computed internally for the 8 most significant bits of red, green and blue channel.

- Two measurements modes are supports

  - Periodic measurements

  - Single measurements

### Reference Check

- In periodic measurement, configured reference signature is compared against with measured signature each frame.

- In case certain no. of consecutive frames with wrong signature, error status is set and optionally interrupt can be issued.

- Error status are reset explicitly by SW or implicitly by HW, when certain number of consecutive frame has correct signature.

### Evaluation windows

- Up to 8 evaluation windows can be setup. Signature computation and reference check is done individually for each window.

- A pixel of the input frame does not contributes to more than one window. In case of overlapping windows a fixed priority of the windows is applied.

- Evaluation windows can be enabled while their signature computation and reference check is disabled. By that they can be used as a skip window only for other evaluation windows.

### Alpha masking

- The pixels considered for signature computation with all evaluation windows can be masked with 1-bit alpha value that the input frame provides for each pixel. By that any kind of shape can be monitored (e.g. see yellow area).

- The mask is considered for checksum computation only, not for assignment of individual pixels to a certain evaluation window. So a non-rectangular overlap between different windows is not possible.

### Panic modes

Panic mode the HW can react to wrong signatures without the need of SW interaction in order to prevent display of corrupted image data.

Two modes are supported

- Local panic

Panic mode the HW can react to wrong signatures without the need of SW interaction in order to prevent display of corrupted image data. Two modes are supported covered by another evaluation window on top without active error. However, it does affect pixels that may be masked out for checksum computation by alpha mask (note, that the alpha is part of the source and may also be wrong then).

- Global panic

When the error status gets active for any evaluation window enabled for this mode, this is signaled to the Frame Generator of the corresponding Display Stream, which can switch to another display mode in response (e.g. switch between Content and Safety Stream or to Constant Color only).

## Safety Features

In order to detect output of corrupt images, multiple signature values can be computed for each frame and checked against reference values.

The signatures can be computed at different points in the display processing path, before or after certain color transformations. This allows to balance between safety aspects and complexity of the reference value determination.

The computation can be limited to rectangular sub areas or to any shaped regions using an alpha bit mask.

In response to a signature violation the following is possible:

• Interrupt signal to SW.

• HW switches autonomously and instantly the monitored region of the corresponding Signature Unit to a constant color.

• HW switches autonomously and instantly the display mode of the Frame Generator.

This allows different scenarios like disabling or enabling certain input streams.

Settings that control these feature are protected against getting modified accidently (See Register Locking). This prevents display of corrupt content in case of major malfunction of a system. These features help to fulfill the requirements of safety standards (e.g. Automotive Safety Integrity Level, ASIL).

## Software Functions

The Signature unit Safe driver provides interfaces to initialize/ un-initialization the signature unit module, control. The following operations will be supported

  - Initialize/ de-initialization the signature unit module

  - Control the signature unit’s evaluation window’s with below feature enablement

    - Enable Local Panic

    - Alpha inverted and mask

    - Enable CRC

    - Enable evaluation window

  - Configure the X,Y coordinates and fix the CRC reference value.

  - Verify the status of the evaluation window.

  - Stop the continuous mode.

## Operating Environment

- Operating System: QNX 7.0.0

- QNX OS for Safety

## Design and Implementation Constraints

Signature unit Safe driver will be provide in a form of static library which has to be linked along with the process using the driver.

## Assumptions and Dependencies

- The Signature unit Safe driver shall be used in the system where QNX OS for Safety package is installed.

- The Signature unit Safe driver shall be used in the system where Mandatory Access Control policy is applied for all the processes in the system.

- The Signature unit Safe driver shall be used in the system where only rootless execution are allowed.

# External Interface

## Hardware Interfaces

- IMX8 config4 board with Daughter board

- Display unit

## Software Interfaces

- The Signature unit Safe driver shall use the APIs provided by the libc library of the QNX OS for Safety package.

## Debug Interfaces

- Tera term.

# Functionality

Three major layer implementation

- Resource manager for signature unit.

- API Library

- Application for sig-window testing.

## Resource Manager for signature unit

### Initialization

Initialize the signature unit module.

Example: (sig0)

| Signature Initialization (display0) |  |  |  |  |
| --- | --- | --- | --- | --- |
| SrcSelect0 | B410h | 0x0 | Tap selection for Signature (display stream 0). Disable framegen#0 for reprogramming. * Select the source is FrameGen#0 output | Before doing initialization, need to do the corresponding signature to unlock (eg. sig0_unlock) Once initialization done, need to lock the signature (eg. sig0_lock). |
| sig0_PanicColor | D00Ch | 0xFFFFFF80 | Overlay color for evaluation windows in panic mode * Enable all RGB color and Alpha mask bit |  |
| sig0_StaticControl | D008h | 0x1 | Global configuration shared by all evaluation windows. * ErrThresReset =0 *ErrThres = 0 *Enables shadow registers for RWS type fields (0 = write through, 1 = shadowed). |  |
| sig0_ContinuousMode | D134h | 0x0 | Signature operation mode control. * EnCont = 0: disables continuous mode. |  |

### Evaluation control and Configuration

Control the signature unit’s evaluation window’s with below feature enablement

| Evaluate window Setup |  |  |  |
| --- | --- | --- | --- |
| sig0_ContinuousMode | D134h | 0x0 | Signature operation mode control. * EnCont = 0: disables continuous mode. |
| sig0_EvalControl0 | D010h | 0x10303 | Control settings for evaluation window 0. To enables the below fields. EnLocalPanic0 : When enabled (value 1) the error status this window (StsSigError) will replace all pixels inside the window by a constant color on the display. Skip regions due to other evaluation windows on top are not modified. AlphaMask, when enabled, is not considered for this replacement. AlphaInv0 : When enabled (value 1) the effect of AlphaMask is inverted (pixels with alpha bit = 1 are ignored then). AlphaMask0 : When enabled (value 1) pixels with alpha bit = 0 are ignored for signature computation. EnCRC0 : When enabled (value 1) the measured signature is checked against a reference value (SigCRCRef). EnEvalWin0 : When enabled (value 1) a CRC signature is computed for all pixels inside this evaluation window (SigCRC). |
| sig0_EvalUpperLeft0 | D014h |  | Upper left corner of evaluation window 0. YEvalUpperLeft0 : Y coordinate. XEvalUpperLeft0 : X coordinate. |
| sig0_EvalLowerRight0 | D018h |  | Lower right corner of evaluation window 0. YEvalLowerRight0 : Y coordinate. XEvalLowerRight0 : X coordinate. |
| sig0_SigCRCRedRef0 | D01Ch |  | Reference signature of red channel for evaluation window 0. |
| sig0_SigCRCGreenRef0 | D020h |  | Reference signature of green channel for evaluation window 0. |
| sig0_SigCRCBlueRef0 | D024h |  | Reference signature of blue channel for evaluation window 0. |
| sig0_ShadowLoad | D130h |  | Shadow load control register. ShdLdReq : Shadow load request for each evaluation window (bit index = window index). |
| sig0_ContinuousMode | D134h | 0x1 | Signature operation mode control. * EnCont = 1: enable continuous mode. |

### Window status and CRC

| Win status/ CRC value |  |  |
| --- | --- | --- |
| Sig0_Status | D13Ch | Read the signature status for the corresponding window, StsSigError : Error status bits for all evaluation windows (bit index = window index). |
| Sig0_SIGCRCRED0 | D028h | CRC values from RED channel. |
| Sig0_SIGCRCGREEN0 | D02Ch | CRC values from GREEN channel. |
| Sig0_SIGCRCBLUE0 | D030h | CRC values from BLUE channel. |

### Shutdown

| Signature de-initialization (display0) |  |  |  |
| --- | --- | --- | --- |
| sig0_ContinuousMode | D134h | 0x0 | Signature operation mode control. * EnCont = 0: disables continuous mode. |
| sig0_Status | D13Ch |  | • Poll StsSigIdle status field until unit is idle (1). StsSigIdle (Bit#20) = 1: Signature is in Idle state (default). StsSigIdle = 0: Signature runs. |

## API Library

Below API’s are support between the Resource manager coupled with BSPAL and Application interface

- dk_bspal_sigunit_initialize(uint8_t sig_unit_id, SSigUnitCfg_t * p_sig_cfg);

- dk_bspal_sigunit_shutdown();

- dk_bspal_sigunit_eval_win_setup(uint8_t win_id, SSigUnitWinCfg_t * p_swconf);

- dk_bspal_sigunit_eval_win_start(uint8_t win_id);

- dk_bspal_sigunit_eval_win_stop(uint8_t win_id);

- dk_bspal_sigunit_get_win_crc_sts( uint8_t win_id);

# Signature Unit BSPAL User Guide

## BSPAL API Usage

| API | Description | Parameters & Return Type |
| --- | --- | --- |
| dk_bspal_sigunit_initialize() | Signature unit BSPAL API to be called in order to initialize the Signature Unit. For further details : Please refer the section 5.1.1 | Parameters : - sig_unit_id - signature unit to initialize. ‘0’ Type : uint8_t - p_sig_cfg – configuration information to pass to initialize signature unit module Type : SSigUnitCfg_t* Return Type : int32_t Returns the status. |
| dk_bspal_sigunit_eval_win_setup() | Signature unit BSPAL API to be called in order to setup the evaluation window. For further details : Please refer the section 5.1.2 | Parameters : - win_id – evaluation window id which user wants to setup (0-7) Type : uint8_t - p_swconf – Evaluation Window configuration to be passed by Application. Refer the configuration in the section 5.1.2 Type : SSigUnitWinCfg_t * Return Type : int32_t Returns the status. |
| dk_bspal_sigunit_eval_win_start() | Signature Unit BSPAL API to be called in order to enable continuous mode on the specific evaluation window For further details : Please refer the section 5.1.2 | Parameters : - win_id – evaluation window id for which user wants to enable continuous mode (0-7) Type : uint8_t Return Type : int32_t Returns the status. |
| dk_bspal_sigunit_eval_win_stop() | Signature Unit BSPAL API to be called in order to disable continuous mode on the specific evaluation window For further details : Please refer the section 5.1.2 | Parameters : - win_id – evaluation window id for which user wants to disable continuous mode (0-7) Type : uint8_t Return Type : int32_t Returns the status. |
| dk_bspal_sigunit_get_win_crc_sts() | Signature Unit BSPAL API to be called in order to get the CRC Status of the specific evaluation window For further details : Please refer the section 5.1.3 | Parameters : - win_id – evaluation window id for which user wants to get the CRC status (0-7) Type : uint8_t Return Type : int32_t Returns the status. |
| dk_bspal_sigunit_shutdown() | Signature unit BSPAL API to be called in order to shut down the Signature Unit. Details regarding Signature Unit shut down. Please refer the section 5.1.4 | void |

| Operation | Commands | Description |
| --- | --- | --- |
| Ethernet configuration | .start_usb_ethernet.sh | start and config the ethernet communication |
|  | SOCK=/sock2 ifconfig |  |
| Export the environment variable | export LIBIMG_CFGFILE=/app/lib/dll/img.conf | Export IMG config for display image |
|  | export LIBRARY_PATH=$LIBRARY_PATH:/app/lib/ | Export LIB path for User build Library |
|  | export LD_IBRARY_PATH=$LD_IBRARY_PATH:/app/lib/ |  |
| Resmgr insmod | resmgr_sigunit -u 0 -v 1 & | insmod the Resource manager to make device sig0 |
| Image display | display_image /app/bin/image.bmp & | Display the image for CRC calculation |
| Application command | sigunit_test -s 0 -w 0 -c "0x0,0x0,0x96,0x96,0x0,0x1,0x0,0x0,0x0" | Run the sig Application to get the CRC value for the above image |

## Copy the binaries and libraries

| Path | Details |
| --- | --- |
| /app/usr/bin/ | display_image |
| /app/bin/ | sig_app, resmgr_sig, image.bmp |
| /app/lib/dll/ | img.conf img_codec_jpg.so , img_codec_tga.so, img_codec_pcx.so, img_codec_tif.so, img_codec_bmp.so, img_codec_png.so, img_codec_wbmp.so, img_codec_gif.so, img_codec_sgi.so |
| /app/usr/lib/ | libimg.so , libimg.so.1 |
| /app/lib/ | libDispSig.so |

## Errors and Fixes

| Error | Fix | Comments |
| --- | --- | --- |
| img_load_file() failed: error accessing dll or entrypoint [8] | For jpg: libjpeg.so (need to be copied the dll/) For png: Libpng.so (need to be copied the dll/) | Use this for debug, Ex: LD_DEBUG=all display_image /app/bin/image2.png |
| bad or missing config file [7] | export LIBIMG_CFGFILE=/app/lib/dll/img.conf | Export configuration path |

## Run Application

| Mode | Command |
| --- | --- |
| Enable Local panic | sigunit_test -s 0 -w 0 -c "0x0,0x0,0x96,0x96,0x0,0x1,0x0,0x0,0x0" |
| Enable Global panic | sigunit_test -s 0 -w 0 -c "0x0,0x0,0x96,0x96,0x0,0x2,0x0,0x0,0x0" |

Application Command line arguments:

-s  :  Signature Unit id.

-w :  Window id.

-c  :   Eval window configuration information value.

- Upper Left x-axis. i.e.0x10

- Upper Left y-axis. i.e.0x10

- Lower Right x-axis. i.e.0xc8

- Lower Right y-axis. i.e.0xc8

- Alpha. i.e. 0 or 1(inverted) or 2(ignored)

- Action i.e. 0 or 1(Local panic) or 2 (Global panic)

- Reference RED CRC value. i.e.0x12345678

- Reference RED CRC value. i.e.0x23456789

- Reference RED CRC value. i.e.0x87654321

# Non Functional Requirements

## Functional Safety Requirements

Response to a signature violation the following is possible:

• Interrupt signal to SW.

• HW switches autonomously and instantly the monitored region of the corresponding Signature Unit to a constant color.

• HW switches autonomously and instantly the display mode of the Frame Generator.

This allows different scenarios like disabling or enabling certain input streams.

## Security Requirements

| Potential Failure Mode | Potential Effects(s) of Failure | Potential Cause(s)/ Mechanism(s) of Failure | Safety Measure |
| --- | --- | --- | --- |
| Image corruption | Image CRC mismatched | Wrong CRC value from the received image | Receive Data Reading Function, check if the CRC is matched with reference value. If a fault is detected then the switch off the display (Global panic)/ or highlighted the particular window with image space (Local panic). |

## Software Quality Attribute

### Maintainability

- Driver shall be provided as a static and shared library package along with the necessary header file to use the defined APIs

- Driver shall follow the coding guidelines specified in Visteon ADAS C Coding Standard - common.xlsm

- Driver shall be analysed using Coverity with MISRA pluings. Any and all deviations if any, shall be approved by the competent architect.

### Portability

- Driver shall be developed using C99 standard

### Reusability

- Driver shall not contain any board specific configurations hardcoded in the driver to enable re-use across other programs based on QNX Operating System with minor modifications.

### Testability

- Driver shall be unit tested with VectorCAST for MC/DC, Statement and Branch Coverage.

- Driver shall be functionally tested for all the supported configuration.

- Driver shall be tested for all the failure modes

# Reference

  - IMX8DQXPRM_Rev_D