Turing C Platform - DoIP

User Guide

Version 1.0

ISSUE DATE: 28 Feb, 2024

Prepared by: Parthiban Chinnathambi  <pchinnat@visteon.com>

______________________________

COC - OS & BSP

PROPRIETARY AND CONFIDENTIAL

This document contains information that is proprietary to Visteon Corporation.    Recipients of the document shall not duplicate, use or disclose information contained herein, in whole or in part, for other than company purposes.

TABLE OF CONTENTS

1	Introduction	4

1.1	Acronyms and abbreviations	4

1.2	References	5

2	Overview	5

2.1	Functional Requirements Satisfied and General Information	5

2.2	Design Information	6

2.2.1 DoIP src and cfg files	6

2.2.2 DoIP doc	6

2.2.3 DoIP  test	6

3 Configuration	6

3.1 DoIP_Cfg.h	6

3.2 DoIP_Lcfg.c	8

3.2.1 const DoIP_ConnectionConfigType DoIP_ConnectionConfig[DOIP_NUM_OF_CONNECTION]	8

3.2.2 const uint8 DoIP_CfgEid[DOIP_EID_LENGTH]	9

3.2.3 const uint8 DoIP_CfgGid[DOIP_GID_LENGTH]	9

3.2.4 const DoIP_TesterConfigType DoIP_TesterConfig[DOIP_NUM_OF_TESTER]	9

3.2.5 const DoIP_RoutingActivationConfigType DoIP_RoutingActivationConfig[DOIP_NUM_OF_ROUTING_ACTIVATION]	9

3.2.6 const DoIP_TcpConConfigType DoIP_TcpConConfig[DOIP_NUM_OF_TCP_CONNECTION]	10

3.2.7 const DoIP_ChannelConfigType DoIP_ChannelConfig[DOIP_NUM_OF_CHANNEL]	10

3.2.8 const DoIP_LocalAddrConfigType DoIP_LocalAddrConfig[DOIP_NUM_OF_LOCAL_ADDR]	10

3.2.9 const DoIP_IpAddrAssignmentConfigType DoIP_IpAddrAssignmentConfig[DOIP_NUM_OF_IP_ADDRESS_ASSIGNEMENT]	11

3.2.10 const DoIP_RxPduConfigType DoIP_RxPduConfig[DOIP_NUM_OF_RX_PDU]	11

3.2.11 const DoIP_DefaultAddrConfigType DoIP_DefaultAddrConfig	11

3.2.12 const DoIP_UserGetFurtherActionByteCallbackFnType DoIP_UserGetFurtherActionByteCallbackFn	11

3.2.13 const DoIP_UserGetPayloadCallbackFnType DoIP_UserGetPayloadCallbackFn	11

3.2.14 const DoIP_UserGetGidCallbackFnType DoIP_UserGetGidCallbackFn	11

3.2.15 const DoIP_UserGetVinCallbackFnType DoIP_UserGetVinCallbackFn	11

3.2.16 const DoIP_UserTriggerGidSyncCallbackFnType DoIP_UserTriggerGidSyncCallbackFn	11

3.2.16 const DoIP_UserGetPowerModeCallbackFnType DoIP_UserGetPowerModeCallbackFn	12

4	Application Usage	12

5	Target System Requirements	12

5.1 Architecture	12

5.2 System Support	12

5.3 System Resource Requirements	12

6	Integration Checklist	13

7. Revision History	14

# 1	Introduction

This document describes basic functionality and the configuration of in-house AUTOSAR Basic Software module DoIP (Diagnostic over IP). Diagnostic over Internet Protocol (DoIP) is a standardized automotive communication protocol used for diagnostics and software updates over IP networks. It enables secure and efficient communication between vehicle Electronic Control Units (ECUs) and diagnostic testers, eliminating the need for physical connections. DoIP utilizes the Internet Protocol (IP) for packet-based communication, allowing diagnostic services to be performed remotely. It supports both Ethernet and IP-based communication, providing flexibility and scalability for modern automotive systems. DoIP ensures robustness, security, and interoperability across different automotive networks, facilitating diagnostics, software updates, and maintenance tasks in vehicles.

## Acronyms and abbreviations

| Term | Description |
| --- | --- |
| Application Layer | The Application Layer is placed above the RTE. Within the Application Layer the AUTOSAR Software-Components are placed. |
| Eth | Ethernet Controller Driver (AUTOSAR BSW module) |
| EthIf | Ethernet Interface (AUTOSAR BSW module) |
| TcpIp | Ethernet State Manager (AUTOSAR BSW module) |
| EthTrcv | Ethernet Transceiver Driver (AUTOSAR BSW module) |
| IP | Internet Protocol |
| TCP | Transmission Control Protocol |
| TCP/IP Stack | Ethernet communication stack |
| VLAN | Virtual Local Area Network |
| MII | Media Independent Interface (standardized Interface provided by Ethernet controllers to access Ethernet transceivers) |
| EC | Ethernet controller |
| ET | Ethernet transceiver |
| BswM | Basic Software Mode Manager |
| ComM | Communication Manager |
| DEM | Diagnostic Event Manager |
| DET | Default Error Tracer |
| EcuM | ECU State Manager |
| SchM | BSW Scheduler |
| SoAd | Socket Adaptor |
| IP | Internet Protocol |
| IPv4 | Internet Protocol version 4 |
| IPv6 | Internet Protocol version 6 |
| MTU | Maximum Transmission Unit |
| TCP | Transmission Control Protocol |
| TCP/IP | A family of communication protocols used in computer networks |
| UDP | User Datagram Protocol |
| ICMP | Internet Control Message Protocol |
| NDP | Neighbor Discovery Protocol |
| DHCP | Dynamic Host Configuration Protocol |
| ARP | Address Resolution Protocol |
| Sd | Service Discovery |
| IF-PDU | An IF-PDU is a PDU which is sent/received via the IF-API of the SoAd TP-PDU |
| TP-PDU | An TP-PDU is a PDU which is send/received via the TP-API of the SoAd |
| EID | Entity identifier |
| GID | Group identifier |
| VIN | Vehicle Identification Number |

## 1.2	References

External to the project

- AUTOSAR_SWS_DiagnosticOverIP.pdf (AUTOSAR version 4.3)

Internal to the project

V-Autosar DoIP package is available at:

- Git folder : https://bsp-os.git.visteon.com/platform/bsp-os/turing/vautosar/com/ethernet

# 2	Overview

## 2.1	Functional Requirements Satisfied and General Information

AUTOSAR functional requirements satisfied in this DoIP can be find at DoIP_SRS.

## 2.2	Design Information

## 2.2.1 DoIP src and cfg files

- DoIP  is implemented in here via DoIP.c in “src” folder. The header files are available in “include” folder

- The template of configuration file can be found in “cfg” as DoIP_Cfg.$h, DoIP_Lcfg.$c , DoIP_Lcfg.$h, DoIP_MemMap.$h , SchM_DoIP.$h, SchM_DoIP_Type.$h   and DoIP_Externals.$h

- The dollar sign should be removed (like DoIP_Cfg.h,) from above template files and used to configure the DoIP as per the program requirements that are located in program specific private configuration folder.

- The DoIP_MemMap should be included in MemMap.h file and the SchM files to be included from the Rte genearation

Please refer the HLD (.Sbsx) for design information

## 2.2.2 DoIP doc

- AUTOSAR_SWS_DoIP.xlsx provides the requirement  mapping of DoIP

- DoIP.sbsx provides the HLD and DD for DoIP component

## 2.2.3 DoIP  test

- Functional Test Plan and Unit Testing plans are stored in test/plans

- Unit testing reports (Aggregate Coverage Report, Full Report, Metrics Report and Testcase Management Report) are stored test/ result

- Vector Cast tool can be run from DOIP_VCAST_HT batch file stored in test\test.harness\DoIP

# 3 Configuration

## 3.1 DoIP_Cfg.h

Configuration items, which used in DoIP_Cfg.h, are listed below:

| S.No | Configuration Item | Description |
| --- | --- | --- |
|  | DOIP_DEV_ERROR_DETECT | Switches the development error detection and notification on or off. |
|  | DOIP_VERSION_INFO_API | Enables / Disables DoIPVersionInfoApi API |
|  | DOIP_MAIN_FUNCTION_PERIOD | Specifies the period of main function DoIP_MainFunction in milliseconds |
|  | DOIP_ALIVE_CHECK_RESPONSE_TIMEOUT | Timeout in [ms] for waiting for a response to an Alive Check request before the connection is considered to be disconnected. |
|  | DOIP_DHCP_OPTION_VIN_USE | If DoIPDhcpOptionVinUse is set to true the DoIP module will add the VIN to the Dhcp host name if no valid Dhcp host name is already set |
|  | DOIP_ENTITY_STATUS_MAX_BYTE_FIELD_USE | This parameter is used to distinguish the optional support of the Max data size element of a diagnostic entity status response |
|  | DOIP_GENERAL_INACTIVITY_TIME | Timeout in [s] for maximum inactivity of a TCP socket connection before the DoIP module will close the according socket connection |
|  | DOIP_GID_INVALIDITY_PATTERN | Specifies the Byte pattern that is used for response messages if no valid GID could be retrieved. Only the value '0' or '255' is allowed |
|  | DOIP_HOST_NAME_SIZE_MAX | Maximum Size of the DHCP HostName in ASCII. |
|  | DOIP_INITIAL_INACTIVITY_TIME | Timeout in [s] used for initial inactivity of a connected TCP socket connection directly after socket connection. |
|  | DOIP_INITIAL_VEHICLE_ANNOUNCEMENT_TIME | Time to wait in [s] for sending first vehicle anouncement message after IP address assignment |
|  | DOIP_MAX_REQUEST_BYTES | Specifies the maximum allowed bytes of a DoIP message request without the DoIP header |
|  | DOIP_MAX_TESTER_CONNECTIONS | Maximum amount of tester connections that shall be maintained at one time before alive check is performed |
|  | DOIP_MAX_UDP_REQUEST_PER_MESSAGE | This parameter captures the maximum amont of UDP Requests necessary to handle paralel within a single UDP connection |
|  | DOIP_NODE_TYPE | Describes the the DoIP node type |
|  | DOIP_USE_EID_AS_GID | Specifies if the DoIP entity shall use its EID if it is the Master for vehicle identification gid on the vehicle identification/vehicle announcement |
|  | DOIP_USE_MAC_ADDRESS_FOR_IDENTIFICATION | Provided the information if a configured EID at vehicle identification response/vehicle announment is used or the MAC address |
|  | DOIP_USE_VEHICLE_IDENTIFICATION_SYNC_STATUS | Defines if the optional VIN/GID synchronization status is used additionally in the vehicle identification/announcement |
|  | DOIP_VEHICLE_ANNOUNCEMENT_COUNT | Number of vehicle announcement messages on IP address assignment |
|  | DOIP_VEHICLE_ANNOUNCEMENT_INTERVAL | Time to wait in [s] for sending subsequent vehicle anouncement messages. |
|  | DOIP_VEHICLE_ANNOUNCEMENT_REPETITION | Amount of repetitions of the vehicle announcement message on IP address assignment. |
|  | DOIP_VIN_GID_MASTER | Specifies if the DoIP entity is the Vehicle identification Master for the GID (Group ID). |
|  | DOIP_VIN_INVALIDITY_PATTERN | Specifies the Byte pattern that is used for response messages if no valid VIN could be retrieved |
|  | DOIP_FURTHER_ACTION_BYTE_CALLBACK_SUPPORT | Direct C Callback function to get the OEM specific Further Action Byte for the DoIP vehicle identification response/vehicle announcement. |
|  | DOIP_GET_GID_CALLBACK_SUPPORT | If the DoIPGetGidDirect parameter exist the DoIP module shall call the configured callback function (<User>_DoIPGetGID) direct. |
|  | DOIP_POWER_MODE_CALLBACK_SUPPORT | If the DoIPPowerModeDirect parameter exist the DoIP module shall call the configured callback function(<User>_DoIPGetPowerModeCallback) direct |
|  | DOIP_TRIGGER_GID_SYNC_CALLBACK_SUPPORT | If the DoIPTriggerGidSyncDirect parameter exist the DoIP module shall call the configured callback function (<User>_DoIPTriggerGidSyncCallback) direct. |
|  | DOIP_LOGICAL_ADDRESS | Describes the logical address of the DoIP entity, i.e. the LA that will route diagnostic requests to the Dcm of the DoIP entity. |
|  | DOIP_OEM_PAYLOAD_TYPE_ENABLED | Defines whether the OEM payload is enabled |
|  | DOIP_GID_VALUE_CONFIGURED | Defines whether the GID value is configured |
|  | DOIP_UDP_MAX_RETRY_COUNT | Defines the maximum number of retry count for UDP |
|  | DOIP_NUM_OF_CONNECTION | Defines the number of DoIP connection supported |
|  | DOIP_NUM_OF_UDP_CONNECTION | Defines the number of UDP connection supported |
|  | DOIP_NUM_OF_TCP_CONNECTION | Defines the number of TCP connection supported |
|  | DOIP_NUM_OF_TCP_UDP_CONNECTION | Defines the number of TCP and UDP connection supported |
|  | DOIP_UDP_NUM_OF_TX_CTRL | Defines the number of UDP Tx supported |
|  | DOIP_NUM_OF_TCP_TX_QUEUE | Defines the number of TCP Tx queue supported |
|  | DOIP_TCP_TX_QUEUE_SIZE | Defines the TCP queue size |
|  | DOIP_NUM_OF_TESTER | Defines the number of Tester supported |
|  | DOIP_NUM_OF_ROUTING_ACTIVATION | Defines the number of routing activation supported |
|  | DOIP_NUM_OF_CHANNEL | Defines the number of channels supported |
|  | DOIP_NUM_OF_LOCAL_ADDR | Defines the number of local address supported |
|  | DOIP_NUM_OF_IP_ADDRESS_ASSIGNEMENT | Defines the number of IP address assignment configuration supported |
|  | DOIP_TCP_RX_MULTI_MSG | Defines whether the Tcp Rx multi message is supported |
|  | DOIP_RX_HEADER_BUFFER_SIZE | Defines the max Rx header buffer size supported |
|  | DOIP_TX_HEADER_BUFFER_SIZE | Defines the max Tx header buffer size supported |
|  | DOIP_TX_IF_BUFFER_SIZE | Defines the Tx If buffer size supported |
|  | DOIP_DIAG_NACK_BUFFER_SIZE | Defines the Diag NACK buffer size supported |
|  | DOIP_OEM_PAYLOAD_UDP_BUFFER_SIZE | Defines the UDP OEM payload buffer size supported |
|  | DOIP_OEM_PAYLOAD_TCP_BUFFER_SIZE | Defines the TCP OEM payload buffer size supported |
|  | DoIP_NUM_OF_TESTER_TO_ROUTING_ACTIVATION | Defines the mapping of tester to routing activation idx |
|  | DOIP_VERIFY_RX_PDU_MAX_DATA_LENGTH | Defines the maximum Rx Pdu verification length |
|  | DOIP_LOCAL_ADDR_TO_CON_MAP_SIZE | Defines the size of local address to Connection mao |
|  | DOIP_RX_PDU_XX | Defines the pdu id for the Rx Pdu |
|  | DOIP_NUM_OF_RX_PDU | Defines the maximum number of Rx Pdu |
|  | DOIP_TX_PDU_DoIPSoAdTcpTxPdu_1 | Defines the pdu id for the Tx Pdu |
|  | DOIP_NUM_OF_TX_PDU | Defines the maximum number of Tx Pdu |

## 3.2 DoIP_Lcfg.c

### 3.2.1 const DoIP_ConnectionConfigType DoIP_ConnectionConfig[DOIP_NUM_OF_CONNECTION]

Defines the configuration for the DoIP connection

| S.No | Element | Description |
| --- | --- | --- |
| 1 | LocalIpAddrIdx | This parameter defines the local address idx supported |
| 2 | RxCancelPduId | This parameter defines the Rx cancel pdu id for this connection |
| 3 | TxPduId | This parameter defines the Tx pdu id for this connection |

### 3.2.2 const uint8 DoIP_CfgEid[DOIP_EID_LENGTH]

Defines the configuration for the EID

| S.No | Element | Description |
| --- | --- | --- |
|  | uint8 | Defines the EID value |

### 3.2.3 const uint8 DoIP_CfgGid[DOIP_GID_LENGTH]

Defines the configuration for the GID

| S.No | Element | Description |
| --- | --- | --- |
|  | uint8 | Defines the GID value |

### 3.2.4 const DoIP_TesterConfigType DoIP_TesterConfig[DOIP_NUM_OF_TESTER]

Defines the configuration for the tester

| S.No | Element | Description |
| --- | --- | --- |
|  | IsRoutingActivationRequired | Defines whether routing activation is required for the tester |
|  | TesterAddr | Defines whether the tester address is required or not for the tester |
|  | NumOfDiagAckNackRespBytes | Defines the number of diag ack/nack response bytes pending for the tester |
|  | RoutingActivationIdx | Defines the routing activation idx for the tester |
|  | DiagNackBufferStartIdx | Defines the Nack buffer start idx for the tester |
|  | DiagNackBufferEndIdx | Defines the Nack buffer end idx for the tester |
|  | TesterToRoutingActivationMapStartIdx | Defines the Tester to Routing activation map start idx for the tester |
|  | TesterToRoutingActivationMapEndIdx | Defines the Tester to Routing activation map end idx for the tester |

### 3.2.5 const DoIP_RoutingActivationConfigType DoIP_RoutingActivationConfig[DOIP_NUM_OF_ROUTING_ACTIVATION]

Defines the configuration for the Routing activation

| S.No | Element | Description |
| --- | --- | --- |
|  | RoutingActivationNumber | Defines the routing activation number for the routing activation |
|  | AuthResLength | Defines the authentication response length for the routing activation |
|  | AuthReqLength | Defines the authentication request length for the routing activation |
|  | ConfReqLength | Defines the confirmation request length for the routing activation |
|  | ConfResLength | Defines the confirmation response length for the routing activation |
|  | ChannelStartIdx | Defines the channel start idx for the routing activation |
|  | ChannelEndIdx | Defines the channel end idx for the routing activation |
|  | pAuthCallbackFn | Defines the authentication callback function the routing activation |
|  | pConfCallbackFn | Defines the authentication confirmation callback function the routing activation |

### 3.2.6 const DoIP_TcpConConfigType DoIP_TcpConConfig[DOIP_NUM_OF_TCP_CONNECTION]

Defines the configuration for the Tcp connection.

| S.No | Element | Description |
| --- | --- | --- |
|  | RxHeaderBufferStartIdx | Defines the Rx Header buffer start idx for the Tcp connection |
|  | TxHeaderBufferStartIdx | Defines the Tx Header buffer start idx for the Tcp connection |
|  | TcpTxQueueElementStartIdx | Defines the Tcp Tx queue element start Idx for the Tcp connection |
|  | TcpTxQueueElementEndIdx | Defines the Tcp Tx queue element end Idx for the Tcp connection |
|  | TcpTxQueueElementSize | Defines the Tcp Tx queue elemnent size for the Tcp connection |

### 3.2.7 const DoIP_ChannelConfigType DoIP_ChannelConfig[DOIP_NUM_OF_CHANNEL]

Defines the configuration for the DoIP channel.

| S.No | Element | Description |
| --- | --- | --- |
|  | IsDefaultChannel | Defines whether this is the default channel |
|  | ApiType | Defines the IF/TP API type supported for this channel |
|  | TesterIdx | Defines the tester idx for this channel |
|  | EcuAddr | Defines the Ecu addr for this channel |
|  | TpMaxLength | Defines the TP max length for this channel |
|  | RxPduId | Defines the Rx Pdu id for this channel |
|  | TxConfId | Defines the Tx confirmation for this channel |
|  | TxIfBufferStartIdx | Defines the Tx IF buffer start idx for this channel |
|  | TxIfBufferSize | Defines the Tx IF buffer size for this channel |

### 3.2.8 const DoIP_LocalAddrConfigType DoIP_LocalAddrConfig[DOIP_NUM_OF_LOCAL_ADDR]

Defines the configuration for the Local address.

| S.No | Element | Description |
| --- | --- | --- |
|  | IsUpdAliveTimeoutEnabled | Defines if the UDP alive timeout enabled for this local address |
|  | IpAddrAssignmentStartIdx | Defines the IP address assignment config start idx for this local address |
|  | IpAddrAssignmentEndIdx | Defines the IP address assignment config end idx for this local address |
|  | ConMapStartIdx | Defines the Connection map start idx for this local address |
|  | ConMapEndIdx | Defines the Connection map end idx for this local address |

### 3.2.9 const DoIP_IpAddrAssignmentConfigType DoIP_IpAddrAssignmentConfig[DOIP_NUM_OF_IP_ADDRESS_ASSIGNEMENT]

Defines the configuration for the Ip address assignment .

| S.No | Element | Description |
| --- | --- | --- |
|  | IsIpAssignementEnabled | Defines if the IP address assignment is enabled for the DoIP |
|  | IpAddrAssignmentType | Defines the IP address assignment type |

### 3.2.10 const DoIP_RxPduConfigType DoIP_RxPduConfig[DOIP_NUM_OF_RX_PDU]

Defines the configuration for the Rx Pdu .

| S.No | Element | Description |
| --- | --- | --- |
|  | ChannelIdx | Defines the channel idx of the Rx Pdu |

### 3.2.11 const DoIP_DefaultAddrConfigType DoIP_DefaultAddrConfig

Defines the configuration for the default address .

| S.No | Element | Description |
| --- | --- | --- |
|  | TesterIdx | Defines the tester idx of the default address |

### 3.2.12 const DoIP_UserGetFurtherActionByteCallbackFnType DoIP_UserGetFurtherActionByteCallbackFn

Defines the configuration of further action byte callback function .

| S.No | Element | Description |
| --- | --- | --- |
|  | DoIP_UserGetFurtherActionByteCallbackFnType | Defines the further action byte callback function |

### 3.2.13 const DoIP_UserGetPayloadCallbackFnType DoIP_UserGetPayloadCallbackFn

Defines the configuration of User payload get callback function .

| S.No | Element | Description |
| --- | --- | --- |
|  | DoIP_UserGetPayloadCallbackFnType | Defines User payload get callback function |

### 3.2.14 const DoIP_UserGetGidCallbackFnType DoIP_UserGetGidCallbackFn

Defines the configuration of User GID get callback function .

| S.No | Element | Description |
| --- | --- | --- |
|  | DoIP_UserGetGidCallbackFnType | Defines User GID get callback function |

### 3.2.15 const DoIP_UserGetVinCallbackFnType DoIP_UserGetVinCallbackFn

Defines the configuration of User VIN get callback function .

| S.No | Element | Description |
| --- | --- | --- |
|  | DoIP_UserGetVinCallbackFnType | Defines User VIN get callback function |

### 3.2.16 const DoIP_UserTriggerGidSyncCallbackFnType DoIP_UserTriggerGidSyncCallbackFn

Defines the configuration of User Trigger GID Sync callback function .

| S.No | Element | Description |
| --- | --- | --- |
|  | DoIP_UserTriggerGidSyncCallbackFnType | Defines User Trigger GID Sync callback function |

### 3.2.16 const DoIP_UserGetPowerModeCallbackFnType DoIP_UserGetPowerModeCallbackFn

Defines the configuration of User Power mode callback function .

| S.No | Element | Description |
| --- | --- | --- |
|  | DoIP_UserGetPowerModeCallbackFnType | Defines User Power mode callback function |

# 4	Application Usage

# 5	Target System Requirements

## 5.1 Architecture

The DoIP does not require any specific hardware system architecture.

## 5.2 System Support

None.

## 5.3 System Resource Requirements

# 6	Integration Checklist

The following checklist to be used by the package user to help insure correctness of package integration and operation.  For each item please check the checkbox upon verification of the item.

| And Code |  |  |
| --- | --- | --- |
| Name and date of persons completing checklist ____________________________________________ |  |  |
| Item # | Item | Checkbox |
| 1 | Check DoIP_Init called before scheduling the DoIP_MainFunction |  |
| 2 | Verify the Eth communication stack are integrated |  |
| 3 | Check DoIP MainFunction called at desired Task Rate |  |

# 7. Revision History

| Revision Level | Section/page | Date | Description | Author |
| --- | --- | --- | --- | --- |
| Version 1.0 | All | 28 Feb 2024 | Initial Release | pchinnat |

ORIGINATOR OF THIS DOCUMENT

pchinnat