Turing C Platform - SoAd

User Guide

Version 1.0

ISSUE DATE: 28 Feb, 2024

Prepared by: Parthiban Chinnathambi  <pchinnat@visteon.com>

______________________________

COC - OS & BSP

PROPRIETARY AND CONFIDENTIAL

This document contains information that is proprietary to Visteon Corporation.    Recipients of the document shall not duplicate, use or disclose information contained herein, in whole or in part, for other than company purposes.

TABLE OF CONTENTS

1	Introduction	4

1.1	Acronyms and abbreviations	4

1.2	References	5

2	Overview	5

2.1	Functional Requirements Satisfied and General Information	5

2.2	Design Information	5

2.2.1 SoAd src and cfg files	5

2.2.2 SoAd doc	6

2.2.3 SoAd  test	6

3 Configuration	6

3.1 SoAd_Cfg.h	6

3.2 SoAd_PBcfg.h	7

3.2 SoAd_PBcfg.c	8

3.2.1 static CONST(SoAd_SoConConfigType, SOAD_CONST) SoAd_SoConConfig[SOAD_NUM_OF_SOCON]	8

3.2.2 static CONST(SoAd_SoConGrpConfigType, SOAD_CONST) SoAd_SoConGrpConfig[SOAD_NUM_OF_SOCON_GROUP]	9

3.2.3 static CONST(SoAd_SocketConfigType, SOAD_CONST) SoAd_SocketConfig[SOAD_NUM_OF_SOCKET]	9

3.2.4 static CONST(SoAd_LocalIpAddrAssignChgFuncType, SOAD_CONST) SoAd_LocalIpAddrAssignChgFuncConfig[SOAD_NUM_OF_LOCAL_IPADDR_ASSIGN_CHG_FUNC]	10

3.2.5 static CONST(SoAd_UpSoConModeChgFuncType, SOAD_CONST SoAd_UpSoConModeChgFuncConfig[SOAD_NUM_OF_UP_SOCON_MODE_CHG_FUNC]	10

3.2.6 static CONST(SoAd_RemAddrIpV4ConfigType, SOAD_CONST) SoAd_RemAddrIpV4Config[SOAD_NUM_OF_REM_ADDR_IPV4]	10

3.2.7 static CONST(SoAd_SocketUdpConfigType, SOAD_CONST) SoAd_SocketUdpConfig[SOAD_NUM_OF_UDP_SOCKET]	10

3.2.8 static CONST(SoAd_SocketTcpConfigType, SOAD_CONST) SoAd_SocketTcpConfig[SOAD_NUM_OF_TCP_SOCKET]	11

3.2.9 static CONST(SoAd_PduRouteConfigType, SOAD_CONST) SoAd_PduRouteConfig[SOAD_NUM_OF_PDU_ROUTE]	11

3.2.10 static CONST(SoAd_PduRouteDestConfigType, SOAD_CONST) SoAd_PduRouteDestConfig[SOAD_NUM_OF_PDU_ROUTE_DEST]	11

3.2.11  static CONST(SoAd_RGPduRouteDestConfigType, SOAD_CONST) SoAd_RGPduRouteDestConfig[SOAD_NUM_OF_RG_PDU_ROUTE_DEST]	12

3.2.12  static CONST(SoAd_RgPduRouteDestIdxType, SOAD_CONST) SoAd_RgPduRouteDestMapConfig[SOAD_NUM_OF_RG_PDU_ROUTE_DEST_MAP]	12

3.2.13  static CONST(SoAd_SocketRouteConfigType, SOAD_CONST) SoAd_SocketRouteConfig[SOAD_NUM_OF_SOCKET_ROUTE]	12

3.2.14  static CONST(SoAd_SocketRouteDestConfigType, SOAD_CONST) SoAd_SocketRouteDestConfig[SOAD_NUM_OF_SOCKET_ROUTE_DEST]	12

3.2.15  static CONST(SoAd_RgSocketRouteDestIdxType, SOAD_CONST) SoAd_RgSocketRouteDestMapConfig[SOAD_NUM_OF_RG_SOCKET_ROUTE_DEST_MAP]	12

3.2.16  static CONST(SoAd_RGSocketRouteDestConfigType, SOAD_CONST) SoAd_RGSocketRouteDestConfig[SOAD_NUM_OF_RG_SOCKET_ROUTE_DEST]	13

3.2.17  static CONST(SoAd_RoutingGroupConfigType, SOAD_CONST) SoAd_RoutingGroupConfig[SOAD_NUM_OF_ROUTING_GROUP]	13

3.2.18  static CONST(SoAd_TpTxBufferConfigType, SOAD_CONST) SoAd_TpTxBufferConfig[SOAD_NUM_OF_TP_TX_BUFFER]	13

3.2.19  static CONST(SoAd_NPduUdpTxConfigType, SOAD_CONST) SoAd_NPduUdpTxConfig[SOAD_NUM_OF_NPDU_UDP_TX]	13

3.2.20  static CONST(SoAd_TcpTxQueueConfigType, SOAD_CONST) SoAd_TcpTxQueueConfig[SOAD_NUM_OF_TCP_TX_QUEUE]	13

3.2.21  static CONST(SoAd_TpRxBufferConfigType, SOAD_CONST) SoAd_TpRxBufferConfig[SOAD_NUM_OF_TP_RX_BUFFER]	14

3.2.22  static CONST(SoAd_RxIfBufferConfigType, SOAD_CONST) SoAd_RxIfBufferConfig[SOAD_NUM_OF_RX_IF_BUFFER]	14

3.2.23  static CONST(SoAd_UpperLayerConfigType, SOAD_CONST) SoAd_UpperLayerConfig[SOAD_NUM_OF_UL]	14

3.2.24  static CONST(SoAd_LocalAddrConfigType, SOAD_CONST) SoAd_LocalAddrConfig[SOAD_NUM_OF_LOCAL_ADDR]	14

3.2.25  static const char SoAd_TcpIpCtrlNameBufferConfig[SOAD_TCPIP_OF_CTRL_NAME_BUFFER_SIZE]	15

3.2.26  static const SoAd_TcpIpCtrlNameIdxConfigType SoAd_TcpIpCtrlNameIdxConfig[SOAD_NUM_OF_TCPIP_CTRL]	15

3.2.27  static const SoAd_SocketTxBufferConfigType SoAd_SocketTxBufferConfig[SOAD_SOCKET_NUM_OF_TX_BUFFER]	15

3.2.28  static const SoAd_SocketTcpRxBufferConfigType SoAd_SocketTcpRxBufferConfig[SOAD_SOCKET_NUM_OF_TCP_RX_BUFFER]	15

4	Application Usage	15

5	Target System Requirements	15

5.1 Architecture	15

5.2 System Support	15

5.3 System Resource Requirements	16

6	Integration Checklist	17

7. Revision History	18

# 1	Introduction

This document describes basic functionality and the configuration of in-house AUTOSAR Basic Software module SoAd (Socket Adaptor). The main purpose of the SoAd module is to create an interface between an AUTOSAR communication service module using PDUs (e.g. PDU Router) and a socket based TCP/IP stack. It will map I-PDU IDs to socket connections and vice versa.

## Acronyms and abbreviations

| Term | Description |
| --- | --- |
| Application Layer | The Application Layer is placed above the RTE. Within the Application Layer the AUTOSAR Software-Components are placed. |
| Eth | Ethernet Controller Driver (AUTOSAR BSW module) |
| EthIf | Ethernet Interface (AUTOSAR BSW module) |
| TcpIp | Ethernet State Manager (AUTOSAR BSW module) |
| EthTrcv | Ethernet Transceiver Driver (AUTOSAR BSW module) |
| IP | Internet Protocol |
| TCP | Transmission Control Protocol |
| TCP/IP Stack | Ethernet communication stack |
| VLAN | Virtual Local Area Network |
| MII | Media Independent Interface (standardized Interface provided by Ethernet controllers to access Ethernet transceivers) |
| EC | Ethernet controller |
| ET | Ethernet transceiver |
| BswM | Basic Software Mode Manager |
| ComM | Communication Manager |
| DEM | Diagnostic Event Manager |
| DET | Default Error Tracer |
| EcuM | ECU State Manager |
| SchM | BSW Scheduler |
| SoAd | Socket Adaptor |
| IP | Internet Protocol |
| IPv4 | Internet Protocol version 4 |
| IPv6 | Internet Protocol version 6 |
| MTU | Maximum Transmission Unit |
| TCP | Transmission Control Protocol |
| TCP/IP | A family of communication protocols used in computer networks |
| UDP | User Datagram Protocol |
| ICMP | Internet Control Message Protocol |
| NDP | Neighbor Discovery Protocol |
| DHCP | Dynamic Host Configuration Protocol |
| ARP | Address Resolution Protocol |
| Sd | Service Discovery |
| IF-PDU | An IF-PDU is a PDU which is sent/received via the IF-API of the SoAd TP-PDU |
| TP-PDU | An TP-PDU is a PDU which is send/received via the TP-API of the SoAd |

## 1.2	References

External to the project

- AUTOSAR_SWS_EthernetSocketAdaptor.pdf (AUTOSAR version 4.3)

Internal to the project

V-Autosar SoAd package is available at:

- Git folder : https://bsp-os.git.visteon.com/platform/bsp-os/turing/vautosar/com/ethernet

# 2	Overview

## 2.1	Functional Requirements Satisfied and General Information

AUTOSAR functional requirements satisfied in this SoAd can be find at SoAd_SRS.

## 2.2	Design Information

## 2.2.1 SoAd src and cfg files

- SoAd  is implemented in here via SoAd.c in “src” folder. The header files are available in “include” folder

- The template of configuration file can be found in “cfg” as SoAd_Cfg.$h, SoAd_Lcfg.$c , SoAd_Lcfg.$h, SoAd_PBcfg.$h, SoAd_PBcfg.$c , SoAd_MemMap.$h , SchM_SoAd.$h, SchM_SoAd_Type.$h

- The dollar sign should be removed (like SoAd_Cfg.h,) from above template files and used to configure the SoAd as per the program requirements that are located in program specific private configuration folder.

- The SoAd_MemMap should be included in MemMap.h file and the SchM files to be included from the Rte genearation

Please refer the HLD (.Sbsx) for design information

## 2.2.2 SoAd doc

- AUTOSAR_SWS_SoAd.xlsx provides the requirement  mapping of SoAd

- SoAd.sbsx provides the HLD and DD for SoAd component

## 2.2.3 SoAd  test

- Functional Test Plan and Unit Testing plans are stored in test/plans

- Unit testing reports (Aggregate Coverage Report, Full Report, Metrics Report and Testcase Management Report) are stored test/ result

- Vector Cast tool can be run from SOAD_VCAST_HT batch file stored in test\test.harness\SoAd

# 3 Configuration

## 3.1 SoAd_Cfg.h

Configuration items, which used in SoAd_Cfg.h, are listed below:

| S.No | Configuration Item | Description |
| --- | --- | --- |
|  | SOAD_DEV_ERROR_DETECT | Switches the development error detection and notification on or off. |
|  | SOAD_GET_AND_RESET_MEASUREMENT_DATA_API | Disables /Enables the measurement data api |
|  | SOAD_IPV6_ADDRESS_ENABLED | Enables/Disables the support of IpV6 |
|  | SOAD_VERSION_INFO_API | Enables and disables the version info API. |
|  | SOAD_MAIN_FUNCTION_PERIOD | Defines the SoAd Mainfunction in milliseconds |
|  | SOAD_ROUTING_GROUP_MAX | Defines the number of routing group supported |
|  | SOAD_SO_CON_MAX | Defines the number of maximum socket connection id supported |
|  | SOAD_MAX_PDU_LENGTH | Defines the maximum pdu length supported by SoAd |
|  | SOAD_TCPIP_API | Defines whether the lower layer TcpIp is AUTOSAR or BSD Linux |
|  | SOAD_SOCKET_ID_INVALID | If SOAD_TCPIP_API is AUTOSAR then this defines the invalid socket id of the AUTOSAR TCPIP |
|  | SOAD_PDU_ PduRoute _XX | Defines the Pdu id of the Pdu Routes which is used by the upper layer |
|  | SOAD_PDU_SocketRoute_XX | Defines the Pdu id of the Socket Routes which is used by the upper layer |
|  | SOAD_RG_XX | Defines the routing group id which is used by the upper layer |
|  | SOAD_SOCON_XX | Defines the Socket connection id which is used by the upper layer |

## 3.2 SoAd_PBcfg.h

Configuration items, which used in SoAd_PBcfg.h, are listed below:

| S.No | Configuration Item | Description |
| --- | --- | --- |
|  | SOAD_NUM_OF_SOCON | The number of socket connection id supported |
|  | SOAD_NUM_OF_SOCON_GROUP | The number of socket connection group supported |
|  | SOAD_NUM_OF_SOCKET | The number of socket supported |
|  | SOAD_NUM_OF_ROUTING_GROUP | The number of routing group supported |
|  | SOAD_NUM_OF_REM_ADDR_IPV4 | The number of remote ipv4 addr supported |
|  | SOAD_NUM_OF_LOCAL_ADDR | The number of local address supported |
|  | SOAD_NUM_OF_TCPIP_CTRL | The number of TcpIp ctrl supported |
|  | SOAD_NUM_OF_LOCALADDR_UNICAST | The number of local addr unicast supported |
|  | SOAD_NUM_OF_LOCAL_IPADDR_ASSIGN_CHG_FUNC | The number of local ip addr assign change function supported |
|  | SOAD_NUM_OF_UP_SOCON_MODE_CHG_FUNC | The number of socket mode change callback function supported |
|  | SOAD_NUM_OF_UL | The number of Upper layer supported |
|  | SOAD_NUM_OF_PDU_ROUTE | The number of Pdu route supported |
|  | SOAD_NUM_OF_PDU_ROUTE_DEST | The number of Pdu route destination supported |
|  | SOAD_NUM_OF_RG_PDU_ROUTE_DEST_MAP | The number of routing group to pdu route destination map supported |
|  | SOAD_NUM_OF_RG_PDU_ROUTE_DEST | The number of routing group to pdu route destination supported |
|  | SOAD_NUM_OF_TX_PDU | The number of Tx pdu supported |
|  | SOAD_NUM_OF_SOCON_TX_PDU_CTRL | The number of Socket connection with tx pdu supported |
|  | SOAD_NUM_OF_SOCKET_ROUTE | The number of socket route supported |
|  | SOAD_NUM_OF_SOCKET_ROUTE_DEST | The number of socket route destination supported |
|  | SOAD_NUM_OF_RG_SOCKET_ROUTE_DEST_MAP | The number of routing group socket route destination map supported |
|  | SOAD_NUM_OF_RG_SOCKET_ROUTE_DEST | The number of routing group to socket route destination supported |
|  | SOAD_NUM_OF_RX_PDU | The number of rx pdu supported |
|  | SOAD_NUM_OF_SOCON_RX_PDU_CTRL | The number of SoCon with Rx pdu support |
|  | SOAD_NUM_OF_UDP_SOCKET | The number of Udp socket supported |
|  | SOAD_NUM_OF_TX_UDP_ALIVE_TIMER | The number of Udp alive timer supported |
|  | SOAD_NUM_OF_NPDU_UDP_TX | The number of Udp socket with NPdu support |
|  | SOAD_NUM_OF_NPDU_UDP_TX_QUEUE | The number of Udp socket with NPdu queue support |
|  | SOAD_NUM_OF_TX_UDP_NPDU_TIMER | The number of Udp socket with NPdu timer support |
|  | SOAD_NUM_OF_TCP_SOCKET | The number of tcp socket supported |
|  | SOAD_NUM_OF_TP_TX_BUFFER | The number of Tp tcp tx buffer supported |
|  | SOAD_TP_TX_BUFFER_SIZE | The size of the Tp tcp tx buffer supported |
|  | SOAD_ROUTING_GROUP_IF_TX_BUFFER_SIZE | The size of the routing group if buffer supported |
|  | SOAD_NUM_OF_TP_RX_BUFFER | The number of the TP Rx buffer supported |
|  | SOAD_TP_RX_BUFFER_SIZE | The size of TP Rx buffer supported |
|  | SOAD_NUM_OF_RX_BUFFER_SEGMENT | The maximum number of Rx buffer segment for a tcp socket . |
|  | SOAD_RX_BUFFER_SEGMENT_DATA_SIZE | Max buffer size to hold the tcp segment |
|  | SOAD_NUM_OF_RX_IF_BUFFER | The number of rx if buffer supported |
|  | SOAD_RX_META_DATA_BUFFER_SIZE | The size of If rx meta data buffer supported |
|  | SOAD_RX_IF_BUFFER_SIZE | The size of the trigger buffer supported |
|  | SOAD_TRIGGER_BUFFER_SIZE | The size of rx if buffer supported |
|  | SOAD_NPDU_UDP_TX_BUFFER_SIZE | The size of Npdu tx buffer supported |
|  | SOAD_NPDU_UDP_TX_QUEUE_RETRY_CNT | The retry count for the Npdu tx queue supported |
|  | SOAD_NUM_OF_TCP_TX_QUEUE | The number of Tcp tx queue supported |
|  | SOAD_TCP_TX_QUEUE_SIZE | The size of the Tcp tx queue |
|  | SOAD_TCPIP_OF_CTRL_NAME_BUFFER_SIZE | the buffer size of the TcpIp ctrl name |
|  | SOAD_SOCKET_LOCAL_ADDR_CHECK_COUNT | the local address check count |
|  | SOAD_SOCKET_TX_BUFFER_SIZE | Defines the total size of Tx buffer for the Bsd socket |
|  | SOAD_SOCKET_NUM_OF_TX_BUFFER | Defines the total size of Tx buffer for the Bsd socket |
|  | SOAD_SOCKET_UDP_RX_BUFFER_SIZE | Defines the total size of Udp Rx buffer for the Bsd socket |
|  | SOAD_SOCKET_TCP_RX_BUFFER_SIZE | Defines the total size of Udp Rx buffer for the Bsd socket |
|  | SOAD_SOCKET_NUM_OF_TCP_RX_BUFFER | Defines the total size of Rx Tcp buffer for the Bsd socket |

## 3.2 SoAd_PBcfg.c

### 3.2.1 static CONST(SoAd_SoConConfigType, SOAD_CONST) SoAd_SoConConfig[SOAD_NUM_OF_SOCON]

Defines the configuration for Socket Connection

| S.No | Element | Description |
| --- | --- | --- |
|  | SoConGrpIdx | Defines the index of SoCon group (SoAd_SoConGrpConfig) |
|  | SoConId | Defines the Socket connection id which is used by other layers |
|  | SocketIdx | Defines the index of socket Id |
|  | RemAddrIpV4Idx | Defines the index of Remote address configuration |
|  | RcvRemAddrIpV4Idx | Defines the index to store the received Remote address |
|  | RemAddrState | Defines the configured remote address state |
|  | PduRouteDestStartIdx | Defines the start id of the PduRouteDest for this SoConId |
|  | PduRouteDestEndIdx | Defines the end id of the PduRouteDest for this SoConId |
|  | SocketRouteStartIdx | Defines the start id of the PduRouteDest for this SoConId |
|  | SocketRouteEndIdx | Defines the end id of the PduRouteDest for this SoConId |
|  | LocalIpAddrAssignmentChgIndStartIdx | Defines the start id of the SoAd_LocalIpAddrAssignmentChgCbkInd for this SoConId |
|  | LocalIpAddrAssignmentChgIndEndIdx | Defines the end id of the SoAd_LocalIpAddrAssignmentChgCbkInd for this SoConId |
|  | SoConModeChgCbkIndStartIdx | Defines the start id of the SoAd_SoConModeChgCbkInd for this SoConId |
|  | SoConModeChgCbkIndEndIdx | Defines the end id of the SoAd_SoConModeChgCbkInd for this SoConId |
|  | SoConCfgOptionMask | Defines the SoCon configuration option bit mask |
|  | TcpTxQueueIdx | Defines the id of the TcpTxQueue for this SoConId |
|  | TpTxBufferCfgIdx | Defines the id of the Tp Tx Buffer Config for this SoConId |
|  | TpRxBufferCfgIdx | Defines the id of the Tp Tx Buffer Config for this SoConId |
|  | SoConTxPduCtrlIdx | Defines the id of the Tx Pdu Ctrl for this SoConId |
|  | SoConRxPduCtrlIdx | Defines the id of the Rx Pdu Ctrl for this SoConId |
|  | NPduUdpTxIdx | Defines the id of the NPdu UDP Tx for this SoConId |
|  | TriggerBufferStartIdx | Defines the start id of the trigger buffer for this SoConId |
|  | TriggerBufferEndIdx | Defines the end id of the trigger buffer for this SoConId |
|  | TriggerBufferSize | Defines the trigger buffer size for this SoConId |
|  | RxBufferSegIdx | Defines the Id of the Rx buffer segment for this SoConId |
|  | RxIfBufferIdx | Defines the Id of the Rx If buffer for this SoConId |

### 3.2.2 static CONST(SoAd_SoConGrpConfigType, SOAD_CONST) SoAd_SoConGrpConfig[SOAD_NUM_OF_SOCON_GROUP]

Defines the configuration for the SoCon Group

| S.No | Element | Description |
| --- | --- | --- |
|  | FramePriority | Defines the frame priority of this SoCon Group |
|  | LocalAddrIdx | Defines the Local address id of this SoCon Group |
|  | SoConStartIdx | Defines the SoCon start id of this SoCon Group |
|  | SoConEndIdx | Defines the SoCon start id of this SoCon Group |
|  | SoConGrpSocketIdx | Defines the SocketId for the TCP SoConGroup |
|  | SocketTcpIdx | Defines the TCP id of this SoCon Group |
|  | SocketUdpIdx | Defines the UDP id of this SoCon Group |
|  | LocalPortOfSoConGrp | Defines the Local port value of this SoCon Group |
|  | SoConCfgGrpOptionMask | Defines the SoCon configuration option bit mask |

### 3.2.3 static CONST(SoAd_SocketConfigType, SOAD_CONST) SoAd_SocketConfig[SOAD_NUM_OF_SOCKET]

Defines the configuration for the Socket

| S.No | Element | Description |
| --- | --- | --- |
|  | SoConStartIdx | Defines the SoCon start id for this Socket Id |
|  | SoConEndIdx | Defines the SoCon end id for this Socket Id |
|  | SoConGrpIdx | Defines the socket connection group id for this Socket Id |
|  | TcpRxBufferEnable | Defines whether Rx buffer is enabled for this socket(bsd only ) |
|  | TcpRxBufferCtrlIdx | Defines the Rx buffer ctrl idx for this socket (bsd only) |
|  | TcpRxBufferIdx | Defines the Rx buffer idx for this socket (bsd only) |

### 3.2.4 static CONST(SoAd_LocalIpAddrAssignChgFuncType, SOAD_CONST) SoAd_LocalIpAddrAssignChgFuncConfig[SOAD_NUM_OF_LOCAL_IPADDR_ASSIGN_CHG_FUNC]

Defines the configuration for the SoAd Local Ip Address Assign Change Function

| S.No | Element | Description |
| --- | --- | --- |
|  | SoAd_LocalIpAddrAssignChgFuncType | Defines the local address assignment change function |

### 3.2.5 static CONST(SoAd_UpSoConModeChgFuncType, SOAD_CONST SoAd_UpSoConModeChgFuncConfig[SOAD_NUM_OF_UP_SOCON_MODE_CHG_FUNC]

Defines the configuration for the SoAd SoCon Mode Change Function

| S.No | Element | Description |
| --- | --- | --- |
|  | UpSoConModeChgFuncType | Defines the SoCon Mode Change Function |

### 3.2.6 static CONST(SoAd_RemAddrIpV4ConfigType, SOAD_CONST) SoAd_RemAddrIpV4Config[SOAD_NUM_OF_REM_ADDR_IPV4]

Defines the configuration for the Ipv4 remote address

| S.No | Element | Description |
| --- | --- | --- |
|  | RemAddrNetMask | Defines the remote address netmask |
|  | RemAddrIpV4Addr | Defines the remote IPv4 address |
|  | RemAddrIpV4Port | Defines the remote port |

### 3.2.7 static CONST(SoAd_SocketUdpConfigType, SOAD_CONST) SoAd_SocketUdpConfig[SOAD_NUM_OF_UDP_SOCKET]

Defines the configuration for the UDP socket

| S.No | Element | Description |
| --- | --- | --- |
|  | SocketUdpChecksumEnabled | Defines whether the UDP checksum calculation in SoAd is enabled or not for this UDP socket |
|  | SocketUdpListenOnly | Defines whether the UDP socket is in Listen only mode.(only Rx) for this UDP socket |
|  | ImmTxConfirmation | Defines whether the immediate confirmation support is enabled or not for this UDP socket |
|  | UdpStrictHeaderLenCheck | Defines whether the strict header length support is enabled or not for this UDP socket |
|  | UdpTriggerTimeout | Defines the UDP trigger timeout value for this UDP socket |
|  | AliveSupervisionTime | Defines the UDP Alive supervision timeout value for this UDP socket |
|  | TxUdpAliveTimerIdx | Defines the UDP Alive supervision timer for this UDP socket |
|  | NPduUdpTxBufferMin | Defines the NPdu UDP Tx buffer minimum length for this UDP socket |
|  | NPduUdpTxQueueSize | Defines the number of NPdu UDP Tx queue for this UDP socket |

### 3.2.8 static CONST(SoAd_SocketTcpConfigType, SOAD_CONST) SoAd_SocketTcpConfig[SOAD_NUM_OF_TCP_SOCKET]

Defines the configuration for the Tcp socket

| S.No | Element | Description |
| --- | --- | --- |
|  | SocketTcpInitiate | Defines whether the TCP connection is initated by SoAd for this TCP socket |
|  | TcpKeepAlive | Defines whether the TCP keep alive enabled or not for this TCP socket |
|  | TcpNoDelay | Defines whether the congestion control mechanism is used for this TCP socket |
|  | TcpRxBufferMin | the minimum Rx buffer size used for this TCP socket |
|  | KeepAliveProbesMax | Defines the Maximum number of times that TCP retransmits an individual data segment before aborting the connection for this TCP socket |
|  | TcpKeepAliveInterval | the time in seconds between the last data packet sent and the first keepalive probe for this TCP socket |
|  | TcpKeepAliveTime | Defines the interval in seconds between subsequent keepalive probes. for this TCP socket |

### 3.2.9 static CONST(SoAd_PduRouteConfigType, SOAD_CONST) SoAd_PduRouteConfig[SOAD_NUM_OF_PDU_ROUTE]

Defines the configuration for the Pdu Route

| S.No | Element | Description |
| --- | --- | --- |
|  | TxConfPduId | Defines the confirmation id of the Pdu Route |
|  | PduRouteULIdx | Defines the upper layer id of the Pdu Route |
|  | PduRouteULType | Defines the upper layer type of the Pdu Route |
|  | PduRouteCfgOptionMask | Defines the config option bit mask of Pdu Route |
|  | PduRouteDestStartIdx | Defines the start id of Pdu Route destination for this Pdu route |
|  | PduRouteDestEndIdx | Defines the end id of Pdu Route destination for this Pdu route |
|  | NumOfPduRouteDest | Defines the number of Pdu Route destination for this Pdu route |
|  | MinimumLength | Defines the minimum length required to transmit on this Pdu route |

### 3.2.10 static CONST(SoAd_PduRouteDestConfigType, SOAD_CONST) SoAd_PduRouteDestConfig[SOAD_NUM_OF_PDU_ROUTE_DEST]

Defines the configuration for the Pdu Route Destination

| S.No | Element | Description |
| --- | --- | --- |
|  | SoConIdx | Defines the SoCon Id of the Pdu route dest |
|  | PduRouteIdx | Defines the Pdu Route Id of the Pdu route dest |
|  | TxUdpTriggerMode | Defines the trigger mode for nPdu Tx mode transmission request always or never |
|  | TxPduHeaderId | Defines the header id of the Pdu route dest |
|  | UdpTriggerTimeout | Defines the UDP trigger timeout to send a nPdu of the Pdu route dest |
|  | RgPduRouteDestMapStartIdx | Defines the start Id of RgPduRouteDestMap of the Pdu route dest |
|  | RgPduRouteDestMapEndIdx | Defines the end Id of RgPduRouteDestMap of the Pdu route dest |

### 3.2.11  static CONST(SoAd_RGPduRouteDestConfigType, SOAD_CONST) SoAd_RGPduRouteDestConfig[SOAD_NUM_OF_RG_PDU_ROUTE_DEST]

Defines the configuration for the Rg – Pdu Route Dest

| S.No | Element | Description |
| --- | --- | --- |
|  | SoConIdx | Defines the SoCon Id |

### 3.2.12  static CONST(SoAd_RgPduRouteDestIdxType, SOAD_CONST) SoAd_RgPduRouteDestMapConfig[SOAD_NUM_OF_RG_PDU_ROUTE_DEST_MAP]

Defines the configuration for the Rg – Pdu Route Dest  map

| S.No | Element | Description |
| --- | --- | --- |
|  | SoAd_RgPduRouteDestIdxType | Defines the the Rg – Pdu Route Dest idx |

### 3.2.13  static CONST(SoAd_SocketRouteConfigType, SOAD_CONST) SoAd_SocketRouteConfig[SOAD_NUM_OF_SOCKET_ROUTE]

Defines the configuration for Socket Route

| S.No | Element | Description |
| --- | --- | --- |
|  | SoConIdx | Defines the SoCon Id for this Socket Route |
|  | RxULPduId | Defines the UL Pdu id of this Socket Route |
|  | SocketRouteULIdx | Defines the upper layer id of the Socket Route |
|  | SocketRouteULType | Defines the upper layer type of the Socket Route |
|  | RxPduHeaderId | Defines the Pdu Header id of this Socket Route |
|  | SocketRouteDestIdx | Defines the socket route destination id of this Socket Route |
|  | SocketRouteCfgOptionMask | Defines the config option mask bits for this socket route |

### 3.2.14  static CONST(SoAd_SocketRouteDestConfigType, SOAD_CONST) SoAd_SocketRouteDestConfig[SOAD_NUM_OF_SOCKET_ROUTE_DEST]

Defines the configuration for Socket Route Destination

| S.No | Element | Description |
| --- | --- | --- |
|  | SoConIdx | Defines the SoCon Id for this Socket Route Destination |
|  | RgSocketRouteDestMapStartIdx | Defines the start Id of RgSocketRouteDestMapfor this Socket Route destination |
|  | RgSocketRouteDestMapEndIdx | Defines the end Id of RgSocketRouteDestMapfor this Socket Route destination |

### 3.2.15  static CONST(SoAd_RgSocketRouteDestIdxType, SOAD_CONST) SoAd_RgSocketRouteDestMapConfig[SOAD_NUM_OF_RG_SOCKET_ROUTE_DEST_MAP]

Defines the configuration for routing group Socket Route Destination map

| S.No | Element | Description |
| --- | --- | --- |
|  | SoAd_RgSocketRouteDestIdxType | Defines routing group Socket Route Destination map idx |

### 3.2.16  static CONST(SoAd_RGSocketRouteDestConfigType, SOAD_CONST) SoAd_RGSocketRouteDestConfig[SOAD_NUM_OF_RG_SOCKET_ROUTE_DEST]

Defines the configuration for routing group Socket Route Destination

| S.No | Element | Description |
| --- | --- | --- |
|  | SoConIdx | Defines the SoCon idx for routing group Socket Route Destination idx |

### 3.2.17  static CONST(SoAd_RoutingGroupConfigType, SOAD_CONST) SoAd_RoutingGroupConfig[SOAD_NUM_OF_ROUTING_GROUP]

Defines the configuration for routing group

| S.No | Element | Description |
| --- | --- | --- |
|  | EnabledAtInit | defines whether the routing group should be enabled at init |
|  | IsTriggerable | defines whether the routing group should be triggerable |
|  | RgPduRouteDestStartIdx | defines the start idx of RgPduRouteDest |
|  | RgPduRouteDestEndIdx | defines the end idx of RgPduRouteDest |
|  | RgSocketRouteDestStartIdx | defines the start idx of RgSocketRouteDest |
|  | RgSocketRouteDestEndIdx | defines the end idx of RgSocketRouteDest |

### 3.2.18  static CONST(SoAd_TpTxBufferConfigType, SOAD_CONST) SoAd_TpTxBufferConfig[SOAD_NUM_OF_TP_TX_BUFFER]

Defines the configuration for Tx Tp buffer

| S.No | Element | Description |
| --- | --- | --- |
|  | TpTxBufferStartIdx | defines whether the routing group should be enabled at init |
|  | TpTxBufferEndIdx | defines whether the routing group should be triggerable |
|  | TpTxBufferSize | defines the start idx of RgPduRouteDest |

### 3.2.19  static CONST(SoAd_NPduUdpTxConfigType, SOAD_CONST) SoAd_NPduUdpTxConfig[SOAD_NUM_OF_NPDU_UDP_TX]

Defines the configuration for  NPdu UDP Tx

| S.No | Element | Description |
| --- | --- | --- |
|  | QueueSupport | Defines whether Queue is supported for this Npu UDP Tx |
|  | SoConIdx | Defines the Socket connection Id for this Npu UDP Tx |
|  | NPduUdpTxQueueStartIdx | Defines the start Id of the Queue for this Npu UDP Tx |
|  | NPduUdpTxQueueEndIdx | Defines the end Id of the Queue for this Npu UDP Tx |
|  | NPduUdpTxBufferStartIdx | Defines the start Id of the buffer for this Npu UDP Tx |
|  | NPduUdpTxBufferSize | Defines the end Id of the buffer for this Npu UDP Tx |
|  | TxUdpNPduTimerIdx | Defines the Timer for this Npu UDP Tx |

### 3.2.20  static CONST(SoAd_TcpTxQueueConfigType, SOAD_CONST) SoAd_TcpTxQueueConfig[SOAD_NUM_OF_TCP_TX_QUEUE]

Defines the configuration for  Tx TCP queue

| S.No | Element | Description |
| --- | --- | --- |
|  | DataStartIdx | Defines the queue data start id Tx TCP queue |
|  | DataEndIdx | Defines the queue data end id Tx TCP queue |
|  | QueueSize | Defines the queue size Tx TCP queue |

### 3.2.21  static CONST(SoAd_TpRxBufferConfigType, SOAD_CONST) SoAd_TpRxBufferConfig[SOAD_NUM_OF_TP_RX_BUFFER]

Defines the configuration for  the Rx Tp buffer

| S.No | Element | Description |
| --- | --- | --- |
|  | TpRxBufferStartIdx | Defines the buffer start id Rx Tp Buffer |
|  | TpRxBufferEndIdx | Defines the buffer end id Rx Tp Buffer |
|  | TpRxBufferSize | Defines the buffer size Rx Tp Buffer |

### 3.2.22  static CONST(SoAd_RxIfBufferConfigType, SOAD_CONST) SoAd_RxIfBufferConfig[SOAD_NUM_OF_RX_IF_BUFFER]

Defines the configuration for  the Rx If Buffer

| S.No | Element | Description |
| --- | --- | --- |
|  | RxIfBufferStartIdx | Defines the buffer start id Rx If Buffer |
|  | RxIfBufferEndIdx | Defines the buffer end id Rx If Buffer |
|  | RxIfBufferSize | Defines the buffer size Rx If Buffer |

### 3.2.23  static CONST(SoAd_UpperLayerConfigType, SOAD_CONST) SoAd_UpperLayerConfig[SOAD_NUM_OF_UL]

Defines the configuration for  the UL

| S.No | Element | Description |
| --- | --- | --- |
|  | UpIfRxIndicationFunc | defines the callback Up_[SoAd][If]RxIndication |
|  | UpIfTriggerTransmitFunc | defines the callback Up_[SoAd][If]TriggerTransmit |
|  | UpIfTxConfirmationFunc | defines the callback Up_[SoAd][If]TxConfirmation |
|  | UpTpCopyRxDataFunc | defines the callback Up_[SoAd][Tp]CopyRxData |
|  | UpTpCopyTxDataFunc | defines the callback Up_[SoAd][Tp]CopyTxData |
|  | UpTpRxIndicationFunc | defines the callback Up_[SoAd][Tp]RxIndication |
|  | UpTpStartOfReceptionFunc | defines the callback Up_[SoAd][Tp]StartOfReception |
|  | SoAd_UpTpTxConfirmationFunc | defines the callback Up_[SoAd][Tp]TxConfirmation |

### 3.2.24  static CONST(SoAd_LocalAddrConfigType, SOAD_CONST) SoAd_LocalAddrConfig[SOAD_NUM_OF_LOCAL_ADDR]

Defines the configuration for  the local address configuration

| S.No | Element | Description |
| --- | --- | --- |
|  | TcpIpCtrlIdx | Defines the TcpIpCtrl Idx of this local address |
|  | AddressType | Defines the address type of this local address |
|  | AssignTriggerType | Defines the address assignment trigger (automatic/manual) of this local address |
|  | DomainType | Defines the Socket domain type |
|  | AssignTypeOfLocalAddr | Defines the assignment type |
|  | AddrIdOfLocalAddr | Defines the local address Id in TcpIp |
|  | RemAddrIpV4Idx | Defines the Ipv4 address configuration idx (bsd only) |

### 3.2.25  static const char SoAd_TcpIpCtrlNameBufferConfig[SOAD_TCPIP_OF_CTRL_NAME_BUFFER_SIZE]

Defines the Eth interface name configuration(bsd only)

| S.No | Element | Description |
| --- | --- | --- |
|  | char | Defines the Eth interface name |

### 3.2.26  static const SoAd_TcpIpCtrlNameIdxConfigType SoAd_TcpIpCtrlNameIdxConfig[SOAD_NUM_OF_TCPIP_CTRL]

Defines the Eth interface name buffer config id for the TcpIp ctrl  (bsd only)

| S.No | Element | Description |
| --- | --- | --- |
|  | CtrlNameBufferStartIdx | Defines the Eth interface name buffer start idx |
|  | CtrlNameBufferEndIdx | Defines the Eth interface name buffer end idx |
|  | CtrlNameBufferSize | Defines the Eth interface name buffer size |

### 3.2.27  static const SoAd_SocketTxBufferConfigType SoAd_SocketTxBufferConfig[SOAD_SOCKET_NUM_OF_TX_BUFFER]

Defines the of socket tx buffer config for the TcpIp ctrl (bsd only)

| S.No | Element | Description |
| --- | --- | --- |
|  | TxBufferStartIdx | Defines the Eth interface name buffer start idx |
|  | TxBufferEndIdx | Defines the socket tx buffer buffer end idx |
|  | TxBufferSize | Defines the socket tx buffer size |

### 3.2.28  static const SoAd_SocketTcpRxBufferConfigType SoAd_SocketTcpRxBufferConfig[SOAD_SOCKET_NUM_OF_TCP_RX_BUFFER]

Defines the buffer config for tcp rx buffer. (bsd only)

| S.No | Element | Description |
| --- | --- | --- |
|  | TcpRxBufferStartIdx | Defines the start Id of the Socket Tcp Rx Buffer |
|  | TcpRxBufferEndIdx | Defines the end Id of the Socket Tcp Rx Buffer |
|  | TcpRxBufferSize | Defines the size of the Socket Tcp Rx Buffer |

# 4	Application Usage

# 5	Target System Requirements

## 5.1 Architecture

The SoAd does not require any specific hardware system architecture.

## 5.2 System Support

None.

## 5.3 System Resource Requirements

# 6	Integration Checklist

The following checklist to be used by the package user to help insure correctness of package integration and operation.  For each item please check the checkbox upon verification of the item.

| And Code |  |  |
| --- | --- | --- |
| Name and date of persons completing checklist ____________________________________________ |  |  |
| Item # | Item | Checkbox |
| 1 | Check SoAd_Init called before scheduling the SoAd_MainFunction |  |
| 2 | Verify the Eth driver, EthSM,EthTrcv,TcpIp and EthIf are integrated |  |
| 3 | Check SoAd MainFunction called at desired Task Rate |  |

# 7. Revision History

| Revision Level | Section/page | Date | Description | Author |
| --- | --- | --- | --- | --- |
| Version 1.0 | All | 28 Feb 2024 | Initial Release | pchinnat |

ORIGINATOR OF THIS DOCUMENT

pchinnat