Turing Platform - mOS

User Guide

Version 1.1

ISSUE DATE: Nov 25, 2022

Prepared by: rsivaku4

______________________________

vAutosar Technology Team

PROPRIETARY AND CONFIDENTIAL

This document contains information that is proprietary to Visteon Corporation.    Recipients of the document shall not duplicate, use or disclose information contained herein, in whole or in part, for other than company purposes.

# 1.Table of Contents

1.	Table of Contents	3

2.	Introduction	4

3.	Acronyms	4

4.	References	4

5.	Overview	5

6.	Configuration Specifications	5

7.     Configuration Assumptions 	6

8.     Building with the mOS code package 	6

8.1     RTC File List	6

8.2     Compilation and Linking	8

9.	Scheduled Functions	8

9.1	OsStart	8

9.1.1     When to Invoke:	8

9.1.2     Rationale:	8

9.2	Activate	8

9.3	De-Activate	8

9.4	Active	8

9.5	Interrupt Service Routine	8

9.5.1     When to Invoke:	8

9.5.2      Category:	8

10.	NV Block Definitions	8

11.	Backup RAM Configurations	8

12.	Critical Dependencies	8

12.1         Dependency to other Components	8

12.2        Dependency to the System	8

13.	Important Constraints and Guidelines to use the mOS	8

14.	Integration Checklist	9

15.	Integration Notes	10

16.	Revision History	11

# 2.Introduction

The intention of this User Guide is to assist the designer & integrator about the mOS         Software component.

It is recommended to understand this Guide prior to any changes in the mOS layer or prior to configure the RTE configurator (e.g. Vector Davinci) in order to avoid confusions & rework. The intention of this document is to provide detailed information within the source files, thereby reducing the amount of change required in this document for individual releases of the package.

This document is stored in RTC ..\ turing.bsw.mos.doc \release (or) in GIT

cluster-platform\turing\vautosar\sys\mos\src\cfg

The operating system serves as a basis for application programs which are independent of each other and provides their environment on a processor. The operating system enables a controlled real-time execution of several processes which appear to run in parallel.

The OSEK operating system provides a defined set of interfaces for the user. These interfaces are used by entities which are competing for the CPU.

# 3.Acronyms

| CDD | Complex Device Driver |
| --- | --- |
| DMA | Direct Memory Access |
| PWM | Pulse Width Modulation |
| PCM | Pulse Code Modulation |
| OS | Operating System |
| ECUM | Ecu Mode Manager |

# 4.References

External to the project

None

Internal to the project

- mOS Software Requirements Specification, mOS_SRS.xls, mOS SSR Consolidation Report stored in RTC..\ turing.bsw.mos.doc\specifications.

- mOS.sbsx (Rhapsody file), stored in RTC ..\ ..\ turing.bsw.mos.doc\design.

- mOS Functional Test Plan, mOS_Functional_Test_Plan.xls, stored in RTC ..\ ..\ turing.bsw.mos.test\plans.

- mOS Unit Test Specification, turing_bsw_mos_UT_Spec.xls, vAutosar Generic Config Test Plan, OS Test Plan20 stored in RTC ..\ turing.bsw.mos.test\plans.

- mOS Coverity Deviation Report, Misra Deviation Report, Cert-C Deviation report, Compiler warning Deviation Report, Critical Code Review, ROM RAM Stack Report stored in RTC ..\ turing.bsw.mos.doc\quality.

- mOS Functional Test Plan Review Checklist, Release Notes, User Guide, OS Concurrent Access Report, SW Code Review Checklist, SW Detailed Design Checklist, SW Unit Test Review Checklist, Revision History stored in RTC ..\ turing.bsw.mos.doc\release.

- mOS VCAST Results, Functional Test Results, UT Manual Test Results, vAutosar Generic Confic Test Result stored in RTC ..\ ..\ turing.bsw.mos.test\results.

- mOS Unit Test Scripts, Headers stored in RTC ..\ ..\ turing.bsw.mos.test\test.harness.

- mOS src, cfg, hal, includes, mak files stored in RTC ..\turing.bsw.mos.

# 5.Overview

This document describes the essential requirements on the AUTOSAR Operating System to satisfy the top-level requirements presented in the AUTOSAR SRS.

In general, operating systems can be split up in different groups according to their characteristics, e.g. statically configured vs. dynamically managed. To classify the AUTOSAR OS, here are the basic features: the OS

is configured and scaled statically

is amenable to reasoning of real-time performance

provides a priority-based scheduling policy

provides protective functions (memory, timing etc.) at run-time

is hostable on low-end controllers and without external resources

This feature set defines the type of OS commonly used in the current generation of automotive ECUs, with the exception of Telematic/Infotainment systems.

# 6.Configuration Specifications

The mOS utilizes

- Configuration Specification and user guide to configure this component can be found in

../turing.bsw.mos/cfg/ mOS_Configurator_v3.9.1$.xls. (or)

.../cluster-platform\turing\vautosar\sys\mos\src\cfg

From the generating tool we have the files listed below.

Generated

1.intvect.c

2.intvect.h

3.intvect_asm.S

4.Os_Cfg.c

5.Os_Cfg.h

6.Os_CfgTypes.h

7.Os_HooksCfg.h

8.Os_IntMemMap.h

9.Os_ResourceCfg.c

10.Os_ResourceCfg.h

11.Os_TrustedServiceCfg.c

12.Os_TrustedServiceCfg.h

13.osStacks.c

14.osStacks.h

15.osStacks.ld

Support for User/Privilege mode access levels. In Os_Cfg.c we have Access level settings for the Applications to take Privilege mode or User mode. The OsApplication sheet in the excel 3.9.1 has a separate column now to assign these levels.

- OS_ACCESS_PRIVILEGED – Supervisor mode.

- OS_ACCESS_UNPRIVILEGED – User mode.

We have list below for manually edited files

Manually Edited

1.Os_CfgMemMap.h

2.Os_MemMap.h

3.OsInternal.c – Empty file

4.OsInternal.h

5.OsScheduleTable_Cfg.c – Only constants

6.OsScheduleTable_Cfg.h

In OsInternal.h we have certain configurable macros and the info are listed below.

#define OS_INTERNAL_FPU_ENABLED      STD_ON

1.The value passed for this macro is STD_ON.

2.If the OS_INTERNAL_FPU_ENABLED is enabled FPU is enabled.

#define MOS_INTERNAL_VECTOR_WRITE    STD_OFF

1.The value passed for this macro is STD_OFF.

2.If the MOS_INTERNAL_VECTOR_WRITE is STD_OFF then write operation is                                                    performed.

#define MOS_API_MPU_SWITCH_ENABLE     STD_ON

1.The value passed for this macro is STD_ON.

2.If the MOS_API_MPU_SWITCH_ENABLE is enabled then MPU switch should be enabled.

In OsInternal.h, we have a section Non-Configurable Parameters. Parameters placed between these sections shouldn’t be changed.

# 7.Configuration Assumptions

- Currently OS_CORE_0_OS_ACCESS_ID is concretely defined as 0 in OsInternal.h based on the below assumption.

- For MPU settings, currently it is assumed that 0th MPU (considered as trusted) container has the complete OS RAM R/W access.

- For more information refer C:\Eclipse\RSIVAKU4-TuringComponentBookshelfRelEngorkspace6\vp.vautosar.safety.doc\02_vAutosar_SafetyWorkProducts\Common\Visteon_SEooC_SafetyManual_vAutosar section 9.3.13.

- For projects using m0p, it is assumed that only supervisor mode is used. User mode will be implemented in upcoming release.

# 8.Building with the mOS Code Package

## 8.1 RTC File List

As per the DI Software Release and Build Process Standard the following Source files must be obtained from the RTC \ turing.bsw.mos\src and RTC \ turing.bsw.mos\includes.

OsAlarm.c

OsApi.c

OsApplication.c

OsCore.c

OsCounter.c

OsError.c

OsEvent.c

OsMain.c

OsMemProtection.c

OsMessage.c

OsMetrics.c

OsRes.c

OsSched.c

OsScheduleTable.c

OsStart.c

OsTask.c

OsTime.c

OsTrap.c

Os.h

Os_Internal_MemMap.h

OsApi.h

OsApplication.h

OsDlt.h

Osek.h

OsError.h

OsMain.h

OsMemProtection.h

OsMessage.h

OsMetrics.h

OsPrvTypes.h

OsScheduleTable.h

OsTrap.h

OsTypes.h

- CPU and core related files should be present in RTC \ turing.bsw.mos\hal.

## 8.2 Compilation and Linking

Ensure that proper interface invocation is done with proper parameters. Please include the header files where ever required.

# 9.Scheduled Functions

No Scheduled Functions for OS.

## 9.1 OsStart

9.1.1 When to Invoke:

To be Invoked by the EcuM.

9.1.2 Rationale:

Classic Autosar 4.2.1 recommendation.

## 9.2 Activate

No Activation

## 9.3 De-Activate

No DeActivation

## 9.4 Active

No Active State – mOS is the operating system and will be active once Os_Start() API is invoked.

## 9.5 Interrupt Service Routine

## 9.5.1 When to Invoke:

System Interrupt – User Configurable

Periodicity – BSP-OS core team need to decide on the Scheduler Tick Time

## 9.5.2 Category:

OS Will handle all the CAT2 ISRs

CAT1 ISRs are not supported in the system.

# 10.NV Block Definitions

NA

# 11.Backup RAM Configurations

NA

# 12.Critical Dependencies

## 12.1 Dependency to other Components

OS will configure and use its own Timer.

At least one TASK need to be configured if Os_Start() is invoked

## 12.2 Dependency to the System

BSP-OS Tasks and Periodicity to be followed as recommended by the vAutosar SIP Task configuration catalog.The availability of this document is WIP.

# 13.Important Constraints to use the mOS

The Constraints to use mOS should be present in the location C:\Eclipse\RSIVAKU4-TuringComponentBookshelfRelEngorkspace6\vp.vautosar.safety.doc\02_vAutosar_SafetyWorkProducts\Common\ Visteon_SEooC_SafetyManual_vAutosar section 3.1.

# 14.Integration Notes

mOS Version: turing_bsw_mos_1.1.58 – user mode support for CM4 and CM7

These Integration Notes applicable upto version 1.1.58.

- In this mOS version, folder structure has been changed and main cmake changes in program are needed as shown below.

Example for VW:

Main cmake path: \programs\vw\my2022\fpkb8\vp-build\vp_program.cmake

Add below two macros as shown in image. Values should be program specific. Please refer the below table.

set (CMAKE_CPU_FAMILY             tv2)

set (CMAKE_CPU_FAMILY_SERIES      tv2c)

- Also, OsInternal.c file should be made empty and update OsInternal.h referring to folder cluster-platform\turing\vautosar\sys\mos\src\cfg\arm\CortexM7 for VW

- Integrate latest mOS tool in path: cluster-platform\turing\vautosar\sys\mos\src\cfg\ mOS_Configurator_v3.9.1$.xlsm

- Also, in Os_Cfg.c we have Access level settings for the Applications to take Privilege mode or User mode. The OsApplication sheet in the excel Tool 3.9.1 has a separate column (Application Access Level) now to assign these levels.

- OS_ACCESS_PRIVILEGED – Privilege mode.

- OS_ACCESS_UNPRIVILEGED – User mode.

- Os_HooksCfg.h – In Tool 3.9.1, under Hooks sheet we have a Text Box to add the Function Prototypes needed for the configuration files.

- Tool 3.9.1 – Tasks sheet Running mode column is removed to directly give access in the Application Access Level column of the osapplication sheet of the tool.

Notes: Family name and Series names are listed down for programs

| S: NO | Program | Variant | CPU Family | Family Series |
| --- | --- | --- | --- | --- |
| 1 | Audi | padicid | tv2 | tv2b |
|  |  | padifbk | imx | imx8 |
| 2 | hd | agent99 | tv2 | tv2b |
| 3 | honda | bev | imx | imx8 |
| 4 | ford | bx755_my2021 | imx | imx8 |
|  |  | s28 | imx | imx8 |
|  |  | u725_my2022 | imx | imx8 |
| 5 | Nissan | p42rprc-cid | tv2 | tv2b |
|  |  | p61qr | tv2 | tv2b |
|  |  | j32v | imx | imx8 |
|  |  | p33a | imx | imx8 |
|  |  | p42rprc | imx | imx8 |
|  |  | P13a | imx | imx8 |
| 6 | Renault | rsa10_1 | tv2 | tv2b |
|  |  | rsa1025 | imx | imx8 |
| 7 | Scania | ddu | tv2 | tv2b |
|  |  | extAmp | tv2 | tv2b |
| 8 | gm | fidm | kinetis | Kinetis |
| 9 | Enfield | alpha_ed1a | imx | imxrt117x |
|  |  | ed1a | imx | imxrt117x |
| 10 | hero | acp | imx | imxrt117x |
|  |  | raa | imx | imxrt117x |
| 11 | fca | wrangler | tv2 | tv2c |
| 12 | Tata | tatagen2ic | tv2 | tv2c |
| 13 | Toyota | 38xd | tv2 | tv2c |
| 14 | VW | fpkb8 | tv2 | tv2c |
| 15 | Jlr | cccm | tv2 | tv2b |

# 15.Integration Checklist

The following checklist is to be used by the package user to help insure correctness of package integration and operation. For each item please check the checkbox upon verification of the item.

| Item # | Item | Checkbox |
| --- | --- | --- |
| 1 | Ensure that the OsInitMemory() function is invoked at system initialization from ECUMExt EcuM_AL_DriverInitZero() |  |
| 2 | Ensure that the OsApi_ShutdownOS() function is invoked at system shutdown. |  |
| 3 | Ensure that the OSStart() is scheduled after OsInitMemory() |  |
| 4 | Ensure that no NULL pointer is passed as parameter for any APIs (unless mentioned in configuration) |  |
| 5 | OsInitMemory() shall be invoked as per ECUM init sequence. |  |
| 6 | Ensure that OsInitMemory() is not invoked more than once in a reset cycle. |  |
| 7 | Ensure that no OS APIs are invoked before OsInitMemory(). |  |
| 8 | Ensure to do all configurations and function invocations properly as mentioned in Section 4. |  |

# 16.Revision History

| Revision | Changes done | Updated By | Updated on |
| --- | --- | --- | --- |
| 1.0 | Initial Version | Sivasujith, Sukumaran(S.) | 07-Mar-2020 |
| 1.1 | Updated the integration Checklist | Yesha Jain | 21-04-2021 |
| 1.2 | MPU config handling assumption | Ramasamy Sivakumar | 24-06-2022 |
| 1.3 | Added Integration notes | Ramasamy Sivakumar | 11-08-2022 |
| 1.4 | Updated user guide | Sravani Pillalamarri | 22-08-2022 |
| 1.5 | Updated assumptions | Ramasamy Sivakumar | 25-11-2022 |
| 1.6 | User Mode Support | Eswaran Ganesan | 28-07-2023 |