Turing B Platform - Amd

Programmers Guide

Version 2.0

ISSUE DATE: December 07, 2021

Prepared by: Emil Dinev (edinev)

______________________________

Visteon Automotive Systems

DI Core Software Group

PROPRIETARY AND CONFIDENTIAL

This document contains information that is proprietary to Visteon Corporation.    Recipients of the document shall not duplicate, use or disclose information contained herein, in whole or in part, for other than company purposes.

Table of Contents

1	Introduction	4

1.1.	Definitions	4

1.2	Acronyms	4

1.3	References	5

2	Overview	5

2.1	Functional Requirements Satisfied and General Information	5

2.2	Design Information	5

3	Configuration	7

3.1	Feature configuration	7

3.2	Measurement configuration	8

4.	Application Usage	10

4.1.	Prerequisites:	10

4.2.	Task Measurement:	10

4.3.	ISR Measurement:	11

4.4.	Runnable Measurement:	11

4.5.	Checkpoint Measurement:	12

4.6.	Marker Measurement:	12

4.7.	Stack Measurement:	12

4.8.	APM Report:	12

5.	Target System Requirements	13

5.1.	Architecture	13

5.2.	System Support	13

5.3.	System Resource Requirements	13

6.	Building with the Amd Code Package	14

6.1.	RTC File List	14

6.2.	Create Configuration Files	14

6.3.	Compilation and Linking	14

7.	Important cautions and guidelines to use the AMD	14

8.	Integration Checklist	15

9.	Revision History	16

# Introduction

This document is intended to provide the information necessary to use the Amd Code Package with the End-item. The Amd package contains Autosar Performance Measurement (APM) for the calculation of CPU load and execution metrics of Tasks, ISRs and Runnables. The intention of this document is to provide detailed information within the source files, thereby reducing the amount of change required in this document for individual releases of the package. This document is stored in RTC ..\di.module.amd.features.doc\release

## Definitions

End-Item - A unique product that is delivered to customers outside the DI software community.

## Acronyms

SDS		Software Design Specification

SRS		Software Requirements Specification

DI		Driver Information

HSIS		Hardware Software Interface Specification

HLD		High Level Design

ETM		Engineering Test Mode

ISR		Interrupt Service Routine

LCD		Liquid Crystal Display

NVM 		Non Volatile Memory

PWM		Pulse Width Modulation

TBD		To Be Decided

OS		Operating System

## References

External to the project

None

Internal to the project

- Apm Software Requirements Specification, Apm_SRS.xls, stored in RTC

..\di.module.amd.features.doc \specification

- Amd Software Design Specification, Amd.sbs (Rhapsody file), stored in RTC

..\di.module.amd.features.doc \design

- Apm Functional Test Plan, Apm_Functional_Test_Plan.xls, stored in RTC ..\di.module.amd.features.test\plans.

- Apm Unit Test Specification, Apm_unit_test_specification.xls, stored in RTC ..\di.module.amd.features.test\plans.

# Overview

## Functional Requirements Satisfied and General Information

Amd contains APM, for real time measurement of CPU load and execution metrics of tasks, interrupts and runnables.

Apm can be configured to take measurement for any Tasks, runnables, functions or ISRs.

Caution!

AMD is not intended for End-Item customer delivery. It is for internal measurements only.

## Design Information

- Define APM_ENABLED macro in Apm_Cfg.h or within the build definitions to use the APM functionalities.

- APM has been divided into several modules for different measurements that can be activated by defining certain macros.

  - APM_ENABLE_ISR_MEASUREMENT

  - APM_ENABLE_RUNNABLE_MEASUREMENT

  - APM_ENABLE_CHECKPOINT_MEASUREMENT

  - APM_ENABLE_STARTUP_MARKERS

  - APM_ENABLE_STACK_MEASUREMENT

  - APM_ENABLE_CRITICALSECTION_MEASUREMENT

- Execution time will be calculated for all configured tasks and ISRs once measurement is allowed from the provided interface.

- CPU load will be calculated.

- Overhead execution time due to task switching or interrupt pre-emption will be measured and calculated.

- APM report generator tool can be used to generate the results.

  - Pre-requisites

    - Read the readme for the APM report generator tool before using it.

  - Inputs

    - Input to the APM Report generator tool shall be a memory dump file of the Apm_CpuLoad structure responsible for storing calculated measurements

  - Outputs

    - APM report generator tool will generate the report for all configured measurements.

Please refer the Software Design Specification (SDS) for design information.

# Configuration

The Apm utilizes:

- Configuration files (Apm_Cfg.c, Apm_Cfg.h)

- Source file(Apm.c, Apm.h, Apm_PrivateCfg.h, Apm_Types.h)

These files contain specific information, which at compile time; will configure the APM for application software for the specific End-Item. These configurations shall be done manually as of now.

## Feature configuration

APM implementation has been divided into different modules responsible for performing different measurements. In this case, they are referred to as Features.

#define APM_ENABLED

Define this macro if the APM functionality is required. This define allows the core functionality for measuring Tasks to be enabled. Without this define APM component will not be compiled and the rest of the Feature defines will not have any effect.

#define APM_ENABLE_ISR_MEASUREMENT

Define this macro to enable the APM to measure ISRs. Without this, the ISR Hooks will do nothing.

#define APM_ENABLE_RUNNABLE_MEASUREMENT

Define this macro to enable the APM to measure Runnables. Without this, the Runnable Hooks will do nothing.

#define APM_ENABLE_CHECKPOINT_MEASUREMENT

Define this macro to enable the APM to measure Checkpoints. Checkpoints measure the absolute time between the Pre and Post Checkpoint Hooks without taking into account any preemptability or switching to interrupt context, unlike Runnables measurement. Without this the Checkpoints Hooks will do nothing.

#define APM_ENABLE_STARTUP_MARKERS

Define this macro to enable the APM to measure Start-up Markers. Without this, the Start-up Marker Hooks will do nothing.

#define APM_ENABLE_STACK_MEASUREMENT

Define this macro to enable the APM to measure stack usage for Tasks and ISRs. In order for the stack usage to be measured for ISRs, APM_ENABLE_ISR_MEASUREMENT will be need to be defined as well.

#define APM_ENABLE_CRITICALSECTION_MEASUREMENT

Define this macro to enable the APM Critical Section measurement. This will calculate the maximum and minimum time spent in Critical Sections by each task.

#define APM_ENABLE_DEBUG

Define this macro to enable the APM debug information. This is an optional define that provides the user with additional information helpful when the APM is being integrated into a project.

## Measurement configuration

#define APM_NUMBER_OF_TASKS

Defines the number of Tasks that are being monitored for measurements. This define will usually point to the definition of number of Tasks within the OS configuration.

#define APM_TASK_MEASUREMENT_ENTRY_COUNT

Defines the number of entries in the measurement buffer allocated for Tasks. In order for the APM component to work correctly, adequate buffer size must be allocated. Total measurement buffer size is composed by summing up all the entry count defines, that have been enabled using the Feature defines.

#define APM_NUMBER_OF_ISRS

Defines the number of ISRs that are being monitored for measurements. This define will usually point to the definition of number of ISRs within the OS configuration.

#define APM_ISR_MEASUREMENT_ENTRY_COUNT

Defines the number of entries in the measurement buffer allocated for ISRs. In order for the APM component to work correctly, adequate buffer size must be allocated. Total measurement buffer size is composed by summing up all the entry count defines, that have been enabled using the Feature defines. Will have effect only if APM_ENABLE_ISR_MEASUREMENT is defined.

#define APM_NUMBER_OF_RUNNABLES

Defines the number of Runnables that are being monitored for measurements.

#define APM_RUNNABLES_MEASUREMENT_ENTRY_COUNT

Defines the number of entries in the measurement buffer allocated for Runnables. In order for the APM component to work correctly, adequate buffer size must be allocated. Total measurement buffer size is composed by summing up all the entry count defines, that have been enabled using the Feature defines. Will have effect only if APM_ENABLE_RUNNABLE_MEASUREMENT is defined.

#define APM_NUMBER_OF_CHECKPOINTS

Defines the number of Checkpoints that are being monitored for measurements.

#define APM_CHECKPOINTS_MEASUREMENT_ENTRY_COUNT

Defines the number of entries in the measurement buffer allocated for Checkpoints. In order for the APM component to work correctly, adequate buffer size must be allocated. Total measurement buffer size is composed by summing up all the entry count defines, that have been enabled using the Feature defines. Will have effect only if APM_ENABLE_CHECKPOINT_MEASUREMENT is defined.

#define APM_NUMBER_OF_STARTUP_MARKERS

Defines the number of Start-up Markers that are being saved.

#define APM_MEASUREMENT_COLLECTION_ALLOCATION_START

Optional define used for allocation of measurement buffer within a memory section, different from the rest of the static variables. This is the starting point of the allocation.

#define APM_MEASUREMENT_COLLECTION_ALLOCATION_END

Optional define used for allocation of measurement buffer within a memory section, different from the rest of the static variables. This is the end point of the allocation.

#define APM_TASK_ID

Defines the ID of the Task that contains the Apm_MainFunction(). Without this the APM measurement algorithm will not work correctly.

#define APM_GPT_FREQ_IN_MHZ

Defines the frequency of the Clock Source (APM_GPT_SRC) used for the measurements in MHz.

#define APM_GPT_SRC

Defines the ID of the Clock Source used for the measurements by APM_GET_CURRENT_TIME().

#define APM_GET_CURRENT_TIME()

Define for getting a timestamp from a defined Clock Source (APM_GPT_SRC) used by the measurement Hooks of the APM. This define will usually point to Gpt get time function by using the ID configured by APM_GPT_SRC. (Ex. Gpt_GetTimeElapsed(APM_GPT_SRC))

# Application Usage

Below are the steps to use the APM for Measurement.

## Prerequisites:

- Enable APM for compilation by defining APM_ENABLED as mentioned in Section 3.

- Configure the Clock Source settings in Apm_Cfg.h file with proper values as mentioned in Section 3.

- Schedule the Apm_MainFunction() in a Task in Rte. The periodicity of the Task shall be properly mentioned in the configuration. Recommended periodicity is 10mS.

- Configure Apm_Init() function to be executed before the OS starts within the system. DriverInitZero or DriverInitOne might be good places.

- Configure the Clock Source to be started and available before the OS starts.

- Pre/Post Task Hooks and Pre/Post ISR Hooks shall be enabled in OS configuration.

- Create an implementation for starting the APM measurement by using the Apm_StartMeasurement() function.

## Task Measurement:

- Configure the Task Measurement settings in Apm_Cfg.h file with proper values as mentioned in Section 3.

- Task Measurement Buffer usage can be approximated by taking into account the number of Tasks measured and the cycle time period of these Tasks.
(Ex. If you have 5 Tasks with a cycle time of 10 mS and the Task executing the Apm_MainFunction() is also with a cycle time of 10 mS, then between the executions of the Apm_MainFunction() there can be a maximum of 10 Entries (5 PreHook and 5 PostHook Entries) within the buffer at any given time, before the Apm_MainFunction() is called to interpret and clear the buffer. This of course is if there are not any other factors)

- Invoke Apm_PreTaskProcess() from within PreTaskHook() function. The parameters shall be the Task ID and Task State available by calling GetTaskID() and GetTaskState() respectively.

- Invoke Apm_PostTaskProcess() from within PostTaskHook() function. The parameters shall be the Task ID and Task State available by calling GetTaskID() and GetTaskState() respectively.

## ISR Measurement:

- Configure the ISR Measurement settings in Apm_Cfg.h file with proper values as mentioned in Section 3.

- Enable the ISR measurements by defining APM_ENABLE_ISR_MEASUREMENT

- ISR Measurement Buffer usage can be approximated by taking into account the number of ISRs. It won’t be so straight forward as was with the Task measurement buffer where you can calculate fairly easily the number of executions.

- Invoke Apm_PreIsrProcess() from within UserPreISRHook() function. The parameter shall be the ISR ID which will be passed by the UserPreISRHook() function.

- Invoke Apm_PostIsrProcess() from within UserPostISRHook() function. The parameter shall be the ISR ID which will be passed by the UserPostISRHook() function.

- ISR nesting is not supported

## Runnable Measurement:

- Configure the Runnable Measurement settings in Apm_Cfg.h file with proper values as mentioned in Section 3.

- Enable the ISR measurements by defining APM_ENABLE_RUNNABLE_MEASUREMENT

- Runnable Measurement Buffer usage can be approximated by taking into account the number of Runnables. This should be similar to the calculation for Task buffer usage.

- Invoke Apm_PreRunnableProcess() before calling the function which you are doing measurements to. Runnable Hooks can be generated using VFB Tracing from DaVinci.

- Invoke Apm_PostRunnableProcess() after calling the function which you are doing measurements to. Runnable Hooks can be generated using VFB Tracing from DaVinci.

- Runnable nesting is not supported. Preemptive Tasks with Runnable measurements is supported.

- This code snipped will NOT work correctly:
void Task1(void)

{

Apm_PreRunnableProcess(1);

Runnable1();

Apm_PostRunnableProcess(1);

}

void Runnable1(void)

{

Apm_PreRunnableProcess(2);

RunnableWithinRunnable();

Apm_PostRunnableProcess(2);

}

## Checkpoint Measurement:

- Configure the Checkpoint Measurement settings in Apm_Cfg.h file with proper values as mentioned in Section 3.

- Enable the Checkpoint measurements by defining APM_ENABLE_CHECKPOINT_MEASUREMENT

- Checkpoint Measurement Buffer usage can be approximated by taking into account the number of Checkpoints. This should be similar to the calculation for Task buffer usage.

- Invoke Apm_PreCheckpointProcess() before calling the function which you are doing measurements to.

- Invoke Apm_PostCheckpointProcess() after calling the function which you are doing measurements to.

## Marker Measurement:

- Configure the Start-up Marker Measurement settings in Apm_Cfg.h file with proper values as mentioned in Section 3.

- Enable the Marker measurements by defining APM_ENABLE_MARKER_MEASUREMENT

- Start-up Markers do not use the measurement buffer, so there is not setting for this.

- Invoke Apm_SaveStartupMarker() when you would like to save a timestamp when you reach a point in the code for the first time. This function can be invoked multiple times without overriding the value saved on the first invocation.

## Stack Measurement:

- Configure the Stack Measurement settings in Apm_Cfg.h file with proper values as mentioned in Section 3.

- Enable the Stack usage measurements by defining APM_ENABLE_STACK_MEASUREMENT

- Stack usage Measurement for ISRs is available only if APM_ENABLE_ISR_MEASUREMENT is defined as well.

- Invoke Apm_SaveStartupMarker() when you would like to save a timestamp when you reach a point in the code for the first time. This function can be invoked multiple times without overriding the value saved on the first invocation.

## 4.8	Critical Section Measurement:

  - In Order to measure the critical sections uses for each task, enable the Critical Section Measurement by defining APM_ENABLE_CRITICALSECTION_MEASUREMENT

  - Update the following hooks in Os_HooksCfg.h in Os Configuration as mentioned below.

    - #define OsCfgCriticalSectionStartHook()       Apm_StartCSMeasurement()

    - #define OsCfgCriticalSectionStoptHook()  	  Apm_EndCSMeasurement()

  - Critical Section Measurements will be calculated and Generated with APM Report.

## APM Report:

- In order to be able to get the measurements saved the calculation process will need to be disabled using the Apm_StopMeasurement() function.

- Once measurement taking has been disabled, the structure containing the saved CPU metrics Apm_CpuLoad has to be exported into a memory dump.

- To convert this data into a more readable and human friendly format a Python script APM parser.py exists that parses the memory dump and saves the data into an excel file that represents the APM Report

# Target System Requirements

## Architecture

The Amd does not require any specific software system architecture.

Please see the release notes in Amd_rln.doc

## System Support

None

## System Resource Requirements

A free running timer of 0.1 us resolution is required.

# Building with the Amd Code Package

## RTC File List

As per the DI Software Release and Build Process Standard the following Source files must be obtained from the RTC \di.module.amd.features\src

Apm.c

Apm.h

Apm_Cfg.c

Apm_Cfg.h

Apm_PrivateCfg.h

Apm_Types.h

## Create Configuration Files

There is no utility specific to the APM. Manual modification has to be done for the configuration. For more details, please refer Section 3 and Section 4.

## Compilation and Linking

Ensure that the macro required for APM usage is enabled. Please add the include path in all required makefiles.

# Important cautions and guidelines to use the AMD

The following procedure needs to be followed by the application/core team to ensure the reliability of the system.

- Ensure that the Apm_Init() function is invoked before calling any Hook function.

- Ensure that the Apm_MainFunction() is configured in an at least 10mS Task.

- Make sure to pass the correct measurement IDs for Pre and Post Hook functionalities.

- Always take a minimum of 20000 samples to take the execution time, if possible.

- Ensure to do all configurations and function invocations properly as mentioned in Section 3 and Section 4.

# Integration Checklist

The following checklist is to be used by the package user to help insure correctness of package integration and operation.  For each item, please check the checkbox upon verification of the item.

| Amd Code |  |  |
| --- | --- | --- |
| Name and date of persons completing checklist ____________________________________________ |  |  |
| Item # | Item | Checkbox |
| 1 | Check the APM_ENABLED macro is defined and the source files are included in the compilation list. |  |
| 2 | Ensure that the correct measurement Feature macros have been defined. |  |
| 3 | Include Apm.h file in the application files. |  |
| 4 | Ensure that adequate buffer size has been allocated for measurements. |  |
| 5 | Ensure the APM Pre and Post Hook functions are invoked with correct measurement IDs. |  |
| 6 | Ensure the measurement types are assigned properly. |  |
| 7 | Ensure that the cautions in the programmer’s guide has been followed. |  |

# Revision History

| Revision Level | Section/page | Date | Description | Author |
| --- | --- | --- | --- | --- |
| Version 1.0 | All | 09 Jun 2016 | Initial Release | Ssebast1 |
| Version 1.1 | 2.2 | 08 Mar 2020 | Formal Release 1.0 | Ssebast1 |
| Version 2.0 | All | 07 Dec 2021 | Release 2.0 | edinev |

ORIGINATOR OF THIS DOCUMENT

Ssebast1