Software Detailed Design Specification

1.2

Table of Contents

1	Predesign Review	3

1.1	Purpose	3

1.2	Public interface (Ctrl_Illumination.c)	3

1.	External Interface: Rte_Read Interface	3

2.	Private Interface: Input_Processing	3

3.	Private Interface: Process_Ctrl_Illumination	4

4.	Private Interface: Illumination_Data_Handle	4

5.	Private Interface: Slicing_control	4

6.	Private Interface: Timing_control	4

7.	Private Interface: Default_Control	5

8.	Private Interface: Off_mode_control	5

1.3  Design choice	5

1.4. File structure	6

# References

| Reference | Document Name | Version |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |

# Definitions

| Term | Abbreviation | Description |
| --- | --- | --- |

# Revision History

| Version | Date | Author | Description of changes |
| --- | --- | --- | --- |
| 1.0 | May-05-2025 | AELANGO6 | This document is the detail design of Illumination – Initial Version |
| 1.1 | May-30-2025 | AELANGO6 | Updated the Input and structure to adapt multiple displays |
| 1.2 | June-30-2025 | AELANGO6 | Slew rate control and determination mechanism in slicing is redefined Illumctrl_Op datatype changed from uint8 to uint16 to handle resolution and slew rates |

# Predesign Review

## Purpose

This document is the detail design methodology for Illumination component.

## Public interface (Ctrl_Illumination.c)

Input structure:

typedef struct

typedef struct

{

UInt8 Illumination_Mode;

UInt8 battery_status;

UInt8 Illumination_Sts;

UInt8 power_mode;

UInt8 step_value;

UInt8 step_direction;

UInt8 switch_type;

UInt16 Derating_Value;

} IllumCtrl_Ip;

Output Structure:

typedef struct

{

UInt16 Illumination_Duty_Out [NO_OF_DUTY];

UInt16 Brightness_Out [NO_OF_BGT];

UInt16 RGB_Dimming_Out [NO_OF_RGB];

UInt16 Bulb_PWM_Out [NO_OF_BULB];

UInt16 LED_PWM_Out [NO_OF_LED];

UInt16 Dial_Out [NO_OF_DIAL];

UInt16 Display_Out [NO_OF_DISPLAYS];

UInt16 Telltale_Out [NO_OF_TELLTALES];

} IllumCtrl_Op;

### External Interface: Rte_Read Interface

| Method | Rte_Read Interface(IllumCtrl_Ip fl_IllumCtrl_Ip) |
| --- | --- |
| Reentrant | No |
| Parameters | IllumCtrl_Ip structure |
| Return | void |
| Description | This Rte Read Function Reads the Input data from the Model component |

### Private Interface: Input_Processing

| Method | Static void Input_Processing(void) |
| --- | --- |
| Reentrant | No |
| Parameters | void |
| Return | void |
| Description | Function call from CmpActive to read the inputs from the model |

### Private Interface: Process_Ctrl_Illumination

| Method | Static void Process_Ctrl_Illumination(void) |
| --- | --- |
| Reentrant | No |
| Parameters | void |
| Return | void |
| Description | This function is act as main function to call the subsequent functions to process the Illumination data such as IlluminationMode check, Switching control,etc |

### Private Interface: Illumination_Data_Handle

| Method | Static void Illumination_Data_Handle(IllumCtrl_Ip l_IllumCtrl_Ip, IllumCtrl_Op_Ptr l_IllumCtrl_Op ) |
| --- | --- |
| Reentrant | No |
| Parameters | IllumCtrl_Ip l_IllumCtrl_Ip, IllumCtrl_Op_Ptr l_IllumCtrl_Op |
| Return | void |
| Description | Function used to identify the switching algorithm based on the switch type from Mdl. |

### Private Interface: Slicing_control

| Method | void Slicing_control(IllumCtrl_Ip l_illumctrl_ip,IllumCtrl_Op_Ptr l_illumctrl_op) |
| --- | --- |
| Reentrant | No |
| Parameters | IllumCtrl_Ip l_illumctrl_ip,IllumCtrl_Op_Ptr l_illumctrl_op |
| Return | void |
| Description | Function is used to control the Illumination outputs based on the slewrates configured in CFG files |

### Private Interface: Timing_control

| Method | Static void Timing_control (IllumCtrl_Ip fl_IllumCtrl_Ip, IllumCtrl_Op Fl_IllumCtrl_Op) |
| --- | --- |
| Reentrant | No |
| Parameters | IllumCtrl_Ip fl_IllumCtrl_Ip, IllumCtrl_Op Fl_IllumCtrl_Op |
| Return | void |
| Description | This function is used to control the timing control of the PWM data. |

### Private Interface: Default_Control

| Method | void Default_control(IllumCtrl_Ip l_illumctrl_ip, IllumCtrl_Op_Ptr l_illumctrl_op) |
| --- | --- |
| Reentrant | No |
| Parameters | IllumCtrl_Ip fl_IllumCtrl_Ip, IllumCtrl_Op Fl_IllumCtrl_Op |
| Return | void |
| Description | Function is used to handle default logic of Illumination – extended to core extension area to handle program specific logic |

### Private Interface: Off_mode_control

| Method | void Off_mode(IllumCtrl_Ip l_illumctrl_ip, IllumCtrl_Op_Ptr l_illumctrl_op) |
| --- | --- |
| Reentrant | No |
| Parameters | IllumCtrl_Ip fl_IllumCtrl_Ip, IllumCtrl_Op Fl_IllumCtrl_Op |
| Return | void |
| Description | Function is used to handle the Illumination Off feature – extended to core extension area to handle program specific logic |

## 1.3  Design choice

#### 1.3.1 Ctrl-Illumination flow:

1.3.2. Class Diagram:

## 1.4. File structure

Bookshelf:
Ctrl_Illumination.c and Ctrl_Illumination.h

Core-extension:

Ctrl_Illumination_Extn.c and Ctrl_Illumination_Extn.h