Signature Unit User Guide

The Signature Generator unit (SIG) can calculate different types of checksums for each input frame of pixel stream data (RGB) within a user-defined evaluation window (i.e. its size and position can be programmed), whereby an additional, separate masking window can be used in conjunction to exclude specific areas of the evaluation window (in effect cutting out specific separate areas of interest).

After the signature generation has been configured and then triggered, a set of signatures are generated. The hardware can then evaluate the correctness of the data by comparing the generated signatures with the reference values.

For Titan (Porsche), signature B check sum is used.

Use  hmi_gfx_mgr02_safety_get_signature_result() to configure the signature table in QSPI.

Include hmi_gfx_mgr02_safety.h file for interface and configuration.

1.0 Signature Unit API:

- Evaluation Window:

Application calls the below interface to set the evalution window size.

GUI_GFX_MGR_ERROR_CODE_T hmi_gfx_mgr02_safety_set_signature_eval_window(GFX_MGR02_SIGNATURE_EVAL_WND_T * p_signature_SP);

- Mask window

Application calls the below interface to set the mask window size

GUI_GFX_MGR_ERROR_CODE_T hmi_gfx_mgr02_safety_set_signature_mask(GFX_MGR02_SIGNATURE_MASK_T * p_signature_SP);

- Reference Threshold

Application calls the below interface to set the threshold reference value.

GUI_GFX_MGR_ERROR_CODE_T hmi_gfx_mgr02_safety_set_signature_ref_threshold(GFX_MGR02_SIGNATURE_REF_THRESHOLD_T * p_signature_SP);

- Signature reference value

Application calls the below interface to set the reference value for different images/sig_id. This interface should be called after calling all the above three interfaces.

GUI_GFX_MGR_ERROR_CODE_T hmi_gfx_mgr02_safety_set_signature_id_reference(GFX_MGR02_SIGNATURE_ID_REF_T * p_signature_id);

- Error Count

Application calls the below interface to get the current error count i.e., no of times reference value and signature does not match

UINT32 hmi_gfx_mgr02_safety_get_signature_error_cnt(void);

- Signature Checksum result

Application calls the below interface to get the signature result after running the signature check for the specified evaluation window. This interface will be used by application to configure the signature table for difference screen in QSPI.

void hmi_gfx_mgr02_safety_get_signature_result(GFX_MGR02_SIGNATURE_RESULT_T * p_signature_result_SP);

  - Configuration:

Below macro  should be defined in hmi_gfx_mgr02.cfg to enable signature unit.

/* Define GFX_MGR02_IRIS_SIG_UNIT_ENABLED to enable Signature Unit             */
#define   GFX_MGR02_IRIS_SIG_UNIT_ENABLED

Below configurations should be in hmi_gfx_mgr02_safety.cfg.

GFX_MGR02_IRIS_SIG_UNIT_ENABLED should be defined if application wants to use the signature unit.

/* If signature error count reaches the GFX_MGR02_SIG_UNIT_THRESHOLD_ERR_CNT, display blank occurs */ 
#define   GFX_MGR02_SIG_UNIT_THRESHOLD_ERR_CNT         

/* No of correct frames which will reset the signature error count to zero */
/* If configured as 0, then no reset of error count happens                */
#define   GFX_MGR02_SIG_UNIT_THRESHOLD_ERR_RST_CNT     

/* MML_GDC_SIG_MASKMODE_NOMASK            = 0x00,      Mask mode is no masking */
/* MML_GDC_SIG_MASKMODE_MASKINSIDE        = 0x01,      Mask mode is mask inside mask window */
/* MML_GDC_SIG_MASKMODE_MASKOUTSIDE       = 0x02       Mask mode is mask outside mask window */

#define   GFX_MGR02_SIG_UNIT_VERTICAL_MASK_MODE         
#define   GFX_MGR02_SIG_UNIT_HORIZONTAL_MASK_MODE