GDT Programmer's Guide

RTC Asset Name: ui-core-gdt

Version 1.1

ISSUE DATE:  17 July 2017

UI Software Group,

Visteon

Table of Contents

1.	Introduction	3

2.	Acronyms	3

3.	Definitions	3

4.	Supported Micro Configurations	4

5.	Overview and general information about the package	4

6.	References	4

7.	File List	4

8.	Build process	6

9.1	RTC File List and Checkout Procedure	6

9.2	Directory Structure	6

9.3	Create/Modify Configuration Files	9

9.4	Auto Generated Files Using GDT	10

9.5	Compilation and Linking	10

10.	Package Configuration	10

10.1	Configuring font.csv file	10

10.2	Configuring hmi_gfx_mgr02.cfg file	11

10.3	Configuring hmi_gfx_mgr02_clut_tables.cfg file	19

10.4	Configuring hmi_gfx_mgr02_fonts.cfg file	20

10.5	Configuring hmi_gfx_mgr02_safety.cfg file	20

10.6	Configuring hmi_gfx_mgr02_warp_tables.cfg file	20

11.	Entry Points	20

12	Application Interfaces	20

13	Target System Requirements	22

13.1	IGDT design and code generation	22

13.2	Target build tools	22

14	Metrics	22

15	Integration Checklist	22

15.1	GDT Project Settings	22

15.2	GDT Project Design	25

15.3	Graphics Manager	27

15.3.1	General (Features/Macros applicable for all platforms)	27

15.3.2	Kepler2 platform	27

16	Package	29

17	Appendix B – Recommended Practices	30

18	Appendix C – Revision History	31

## Introduction

This programmer’s guide contains the information on how to configure and manage the ui.core.gdt package software to incorporate inside an End-Item system software.

## Acronyms

| AA | Anti-Alias |
| --- | --- |
| BWC | Band Width Control |
| CLUT | Color Lookup Table |
| CS | Critical Section |
| DAR | Dirty Area Region |
| DCU | Display Controller Unit |
| DI | Driver Information |
| DMA | Direct Memory Access |
| FT2 | Free Type 2 |
| GDT | Graphics Design Tool |
| GfxMgr | Graphics Manager |
| GRAM | Graphics RAM |
| HMI | Human Machine Interface |
| IGDT | Integrated Graphics Design Tool |
| ISR | Interrupt Service Routine |
| LSH | Logic State Handler |
| OVG | OpenVG |
| QSPI | Quad Serial Peripheral Interface |
| RLE | Run Length Encoding |
| TBC | To be confirmed later |
| TCOUNT | Timer Count |
| TFT | Thin Film Transistor |

## Definitions

- Graphics Manager: Software library that manages and controls all user interface components.

- Graphics Design Tool: Software design tool to create user interface screens and its components. This tool also supports code generation from the design.

- Freetype2: It is a popular software development library used to render text onto bitmaps, and provides support for other font-related operations.

- Monotype: Another software library provides support for font related operations.

- OpenVG: It is an API designed for hardware-accelerated 2D vector graphics.

- OpenGL: Open Graphics Library (OpenGL) is a cross-language, cross-platform application programming interface (API) for rendering 2D and 3D vector graphics. The API is typically used to interact with a graphics processing unit (GPU), to achieve hardware-accelerated rendering.

## Supported Micro Configurations

| SNo | Vendor | Micro Controller | Compiler/IDE |
| --- | --- | --- | --- |
| 1 | Renesis | RH850 | GHS |
| 2 | FreeScale | Vybrid (Cortex A5/M4), MPC56455 (Rainbow), MP5606 (Spectrum), | IAR |
| 3 | Spansion | Amber P2, Titan | GHS 6.1.6 GHS 5.2.4 |

## Overview and general information about the package

This package provides the complete UI framework including IGDT, the tool to design and generate code.

## References

Please refer the documents in \ui.core.gdt.doc\hmi.gfx.manager.02\design\

## File List

- Config files:

- font_csv.$cfg (can be used for all platforms)

- hmi_gfx_mgr02.$cfg (can be used for all platforms)

- hmi_gfx_mgr02_clut_table.$cfg (can be used for DCU platform only)

- hmi_gfx_mgr02_font.$cfg (can be used for all platforms)

- hmi_gfx_mgr02_safety.$cfg(can be used for Amber platform only)

- hmi_gfx_mgr02_warp_tables.$cfg(can be used for Amber platform only)

- panel_definition.$h (can be used for Amber platform only)

- hmi_logic_state_handler_template.$c(Generated by IGDT)

- Header files:

- hmi_gfx_mgr02_amber.h

- hmi_gfx_mgr02_amber_iris.h

- hmi_gfx_mgr02_context.h

- hmi_gfx_mgr02_dcu.h

- hmi_gfx_mgr02_defs.h

- hmi_gfx_mgr02_dirty_area.h

- hmi_gfx_mgr02_driver.h

- hmi_gfx_mgr02_generic.h

- hmi_gfx_mgr02_gsub.h

- hmi_gfx_mgr02_irislib.h

- hmi_gfx_mgr02_layout.h

- hmi_gfx_mgr02_mem.h

- hmi_gfx_mgr02_ogllib.h

- hmi_gfx_mgr02_ovglib.h

- hmi_gfx_mgr02_perflog.h

- hmi_gfx_mgr02_safety.h

- hmi_gfx_mgr02_swrlib.h

- hmi_gfx_mgr02_test.h

- hmi_gfx_mgr02_text.h

- hmi_gfx_mgr02_theme.h

- hmi_gfx_mgr02_types.h

- hmi_gfx_mgr02_driver_mqx.$h

- hmi_gfx_mgr02_ft2mem_config.$h

- hmi_gfx_mgr02_amber_mem_mgmt.h

- hmi_gfx_mgr02_monolib.h

- hmi_gfx_driver_cp_image.inc

- C files:

- hmi_gfx_mgr02_amber.c

- hmi_gfx_mgr02_amber_iris.c

- hmi_gfx_mgr02_context.c

- hmi_gfx_mgr02_dcu.c

- hmi_gfx_mgr02_dirty_area.c

- hmi_gfx_mgr02_driver.c

- hmi_gfx_mgr02_generic.c

- hmi_gfx_mgr02_gsub.c

- hmi_gfx_mgr02_irislib.c

- hmi_gfx_mgr02_layout.c

- hmi_gfx_mgr02_mem.c

- hmi_gfx_mgr02_ogllib.c

- hmi_gfx_mgr02_ovglib.c

- hmi_gfx_mgr02_perflog.c

- hmi_gfx_mgr02_safety.c

- hmi_gfx_mgr02_swrlib.c

- hmi_gfx_mgr02_text.c

- hmi_gfx_mgr02_theme.c

- hmi_gfx_mgr02_driver_mqx.$c

- hmi_gfx_mgr02_ft2mem_config.$c

- hmi_gfx_mgr02_amber_mem_mgmt.c

- hmi_gfx_mgr02_monolib.c

- Directories

- amber (Contains Amber platform related files and libraries)

- fsl (Contains FreeScale related files)

- iris (Contains Iris driver files)

- iType (Contains iType lib source files)

- GDT generated files:

- hmi_gfx_anim_engine_cfg.h

- hmi_gfx_anim_engine_data.rom

- hmi_gfx_mgr02_cfg.h

- hmi_gfx_mgr02_clut_table.c

- hmi_gfx_mgr02_clut_table.h

- hmi_gfx_mgr02_code_gen.txt

- hmi_gfx_mgr02_config.dat

- hmi_gfx_mgr02_font.c

- hmi_gfx_mgr02_font.h

- hmi_gfx_mgr02_image_data.h

- hmi_gfx_mgr02_ovg_table.dat

- hmi_gfx_mgr02_svg_table.dat

- hmi_gfx_mgr02_vgfont.c

- hmi_logic_state_handler.dat

- hmi_logic_state_handler.inc

- hmi_logic_state_handler_template.$c

- New_RAM_optimization_report.txt

- gresfiles\Images*.dat

- font*.c

## Build process

## RTC File List and Checkout Procedure

- The files mentioned in the File List section above must be checked out of the folder from the RTC Component Stream ui-core-gdt into product specific sub-directory, where the application source files are stored.

- Configure the necessary .cfg files using the guide lines below.

- Generate the files using GDT tool.

## Directory Structure

ui.core.gdt configuration files path

ui.core.gdt source folder path in RTC:

ui.core.gdt GDT tool path in RTC

## Create/Modify Configuration Files

- Using the file font_csv$.cfg as a template, create a file named font_name.csv and configure using the section Configuring fonts.csv file below as a guide.

- Using the file hmi_gfx_mgr02$.cfg as a template, create a file named

hmi_gfx_mgr02.cfg and configure using the section Configuring hmi_gfx_mgr02.cfg file below as a guide.

- Using the file hmi_gfx_mgr02_clut_table$.cfg as a template, create a file named

hmi_gfx_mgr02_clut_table.cfg and configure using the section Configuring hmi_gfx_mgr02_clut_table.cfg file below as a guide.

- Using the file hmi_gfx_mgr02_font$.cfg as a template, create a file named

hmi_gfx_mgr02_font.cfg and configure using the section Configuring hmi_gfx_mgr02_font.cfg file below as a guide.

- Using the file hmi_gfx_mgr02_safety$.cfg as a template, create a file named

hmi_gfx_mgr02_safety.cfg and configure using the section the Configuring hmi_gfx_mgr02_safety.cfg file as a guide.

- Using the file hmi_gfx_mgr02_warp_tables$.cfg as a template, create a file named hmi_gfx_mgr02_warp_tables.cfg and configure using the section Configuring hmi_gfx_mgr02_warp_tables.cfg file as a guide.

## Auto Generated Files Using GDT

Use the GDT to design the UI and generate code. GDT user guide can be available in \ui.core.gdt.doc\hmi.gfx.manager.02\design\.

## Compilation and Linking

The application specific build files must compile or assemble the required ui.core.gdt package files. Ensure that the sub-directory into which the source code files were downloaded from RTC is on the search path for the compilation process.

## Package Configuration

## Configuring font.csv file

- Copy the sample file font_csv$.cfg to font_name.csv. Do not attempt to change the macro names, use as it is.

- Font file name should be same as the GDT font element name.

- IGDT will parse and render the details in this file and ignore the actual properties of the font element configured in IGDT.

- This file configuration should be used when user wants to configure different font files / font heights under a single GDT font element name.

- The following are the macros and its definitions.

- Example configuration file:

| Macro | Definitions |
| --- | --- |
| NAME | Specify the font name |
| POINT | Specify appropriate floating point value |
| HEIGHT | Specify the font height as integer value |
| STYLE | Specify the font style as either Normal, Bold, Italic, Underline or strikeout |
| PRERENDER | Specify whether pre-render should be enabled or not by setting true or false |
| USE_BGCOLOR | Specify the use background colour as integer value |
| CHAR_SPACING | Specify the character spacing as integer value |
| SPACE_CHAR_WIDTH | Specify the space character width as integer value |
| CHAR_SPACE_CTRL | Specify whether the space control for characters should be enabled or not by giving a Boolean value |
| AA_MODE | Specify the mode as 1BPP/2BPP/4BPP. (or) 1BPPV/2BPPV/4BPPV when vertical packing is enabled |
| AUTO_DETECT_FONT_HEIGHT | Specify whether auto detection of font height should be enabled or not by setting true or False |
| START_OFFSET | Specify the appropriate offset value to be set when auto detect font height is enabled |
| DEFAULT_CHAR | Specify the default char to be displayed if the Unicode range is not specified within RANGE_START to RANGE_END |
| IMAGE_TO_CHAR | Specify whether the image to char should be enabled or disabled by setting true or false |
| IMG_PATH | Specify the image path or null as default |
| text_rendering_hint | Specify any one of the below. SystemDefault: This will take the default settings of the PC. SingleBitPerPixelGridFit: Each character is drawn using its glyph bitmap. Hinting is used to improve character appearance on stems and curvature. SingleBitPerPixel: Each character is drawn using its glyph bitmap. Hinting is not used. AntiAliasGridFit: Each character is drawn using its antialiased glyph bitmap with hinting. Much better quality due to antialiasing, but at a higher performance cost. AntiAlias: Each character is drawn using its antialiased glyph bitmap without hinting. Better quality due to antialiasing. Stem width differences may be noticeable because hinting is turned off. ClearTypeGridFit: Each character is drawn using its glyph ClearType bitmap with hinting. The highest quality setting. Used to take advantage of ClearType font features. |
| font_engine | Specify the font engine as Windows or Freetype2 or Monotype |
| font_file | Specify the font file |
| auto_hint | Enable or disable the auto_hint by setting true or false |
| RANGE_START | Specify the valid range of Unicodes |
| RANGE_END |  |

## Configuring hmi_gfx_mgr02.cfg file

- Copy the sample file hmi_gfx_mgr02$.cfg to hmi_gfx_mgr02.cfg. Do not attempt to change, use as it is.

- The following are the macros and its definitions.

DCU target:

| Macro | Definitions |
| --- | --- |
| GFX_MGR02_NUM_DCU_LAYERS | Number of layers supported by the DCU For rainbow/spectrum - 16 Faraday - 64 |
| GFX_MGR02_RESERVE_LAYERS_ FOR_HIGH_PRIORITY_SCREEN | Generally the layer allocation happens from low priority screen to high priority screen. So there are chances that a high priority screen may not get enough layers to display its content. Define this macro if layers needs to be reserved for high priority screen so that a high priority screen get displayed regardless of layer availability. |
| GFX_MGR02_DCU_BLEND_ITER | Number of layers to blend. Range is 2 - 4 for Rainbow/Spectrum (default is 4) 2 - 6 for Faraday (default is 6) Define this macro other values than default is required. (more information, refer DCU_MODE register description On item BLEND_ITER). |
| GFX_MGR02_ALLOWED_OVERLAP _ERROR_W GFX_MGR02_ALLOWED_OVERLAP _ERROR_H | Specify overlap error allowed when determining an element is hidden below another one. Below configurations allow a layer to be set as hidden even though GFX_MGR_ALLOWED_OVERLAP_ERROR_W pixels AND/OR GFX_MGR_ALLOWED_OVERLAP_ERROR_H Pixels are still visible over the edges. |
| GFX_MGR02_GRAM_BASE GFX_MGR02_GRAM_SIZE | Specifies the base address for the GRAM how much GRAM is available for the DCU target to perform widget and text element building. Below sample configuration are provided for spectrum/rainbow-lite/rainbow assuming full gram is available. This could be changed accordingly by the app need Note: GFX_MGR02_GRAM_BASE should not be set for a WIN32 or MATLAB build. |
| GFX_MGR02_PERFORM_ QSPI_CHECKSUM | The QSPI devices are normally dedicated to DCU and the DCU will be displaying content out of the QSPI memory during active frame period. The only time the DCU will not be accessing it will be during the horizontal and vertical blanking period of the TFT. The checksum calculation of QSPI memory needs the CPU to access it. But allowing it during the DCU is performing the access might result in bandwidth reduction and DCU getting starved of data and ultimately the display losing sync momentarily. |
| GFX_MGR02_CALC_QSPI_CHKSUM | Graphics manager provides a call-back from the vsync ISR where in the QSPI checksum component can safely access the QSPI memory. It is to hook an application checksum calculation function which will be scheduled from vsync ISR. And also to warn that it is called from an ISR and execution times to be kept minimal. This can be enabled by defining GFX_MGR02_CALC_QSPI_CHKSUM. |
| GFX_MGR02_DYNAMIC _CLUT_LOAD_SUPPORT_NEEDED | As you know the DCU has only limited CLUT RAM (8Kbytes for Rainbow). This can hold 2048 colour entries. Your graphics design can result in two situations Situation1 :- The number of CLUT entries is <= 2048colors Situation2 :- The number of CLUT entries is > 2048colors In situation 1, we can load all of the CLUT entries in to CLUT RAM during the start-up and there after the graphics manager need not worry about programming the CLUT table at all. This will be most efficient way since we don't need to spend valuable CPU time in Vsync ISR to program the CLUT RAM. But complex applications always result in CLUT entries > 2048 and in this case we cannot load all of the CLUT entries in to CLUT RAM during start-up. In order to support these two cases, graphics manager supports two ways of CLUT loading. GFX_MGR02_DYNAMIC_CLUT_ LOAD_SUPPORT_NEEDED determines which method should be applied. For situation 1: - You can keep this macro commented out so that graphics manager loads all the CLUT in to CLUT RAM during start up. Uncommenting the macro also will work fine for this situation but is not recommended. For situation 2: - Uncomment or define this macro. This will make the graphics manager to dynamically load the CLUT tables for the images that are on the display/TFT at a given instance. Commenting out this macro in this situation will make images to appear incorrectly who's CLUT are > 2048 mark |
| GFX_MGR02_DCU_ENABLE_HCLIP | define this macro to enable horizontal clipping support |
| GFX_MGR02_DEDICATED_ WORK_BUFFER_SIZE | define the work buffer size in bytes (Recommended to configure this macro, if have external RAM and huge graphics content) |
| GFX_MGR02_DEDICATED_ WORK_BUFFER_BASE | define the work buffer base address in case GFX_MGR02_DEDICATED_WORK_BUFFER_SIZE set > 0 |
| GFX_MGR02_SYNCHRONISE _DMA_BUILD_WITH_VBLANK | Define this macro to synchronize the eDMA based build routines to begin its operations during the vblank period for more information please refer hmi_gfx_mgr02.cfg file |
| GFX_MGR02_BG_DMA _SEQ_RENDERING | All eDMA based rendering is completed and then background built layers are rendered. This ensures lesser memory bandwidth but slightly more rendering time set GFX_MGR02_BG_DMA_SEQ_RENDERING to 1 The background content building is triggered and happens in parallel with eDMA rendering. More bandwidth usage but completes frame building quicker. Set GFX_MGR02_BG_DMA_SEQ_RENDERING to 0 |
| GFX_MGR02_GRAM_ OVERAP_HANDLING | When the GRAM allocation happens for the next updates, there may be a chance that the next updates requires more GRAM and may overlap with the currently active/displayed contents’ GRAM for more info refer .cfg file |
| GFX_MGR02_DMA_CHANNEL | Specify the DMA channel to be used by the Graphics Manager for its building operations. |
| GFX_MGR02_SCHED_ ANIM_TICKS_IN_VSYNC | If defined then schedules the animation ticks from VSYNC ISR. If not defined scheduling the animation ticks (function hmi_gfx_anim_engine_tick()) is left to application control |
| GFX_MGR02_HORIZONTAL_SCROLL_ENABLED | In Kepler platform when rotation is disabled and text scroll is to be performed horizontally from left to right then define the value of macro GFX_MGR02_HORIZONTAL_SCROLL_ENABLED as 1 |
| GFX_MGR02_PROD _SPEC_DISPLAY_SETUP | If you are not using LQ042 display(s) or would like different timing parameters, comment out "GFX_MGR02_USE_SHARP_LQ042" and Define. For more details refer .cfg file |
| GFX_TCOUNT | To a function or timer counter register to read the current free running tick count. |
| GFX_TCOUNT_TYPE | Data type of the tick counter UINT16 or UINT32 for example |
| GFX_TCOUNT_HZ | Frequency in Hz at which the tick counter increments |
| GFX_MGR02_USE_SHARP_LQ042 | If you are using the Sharp LQ042 display connected to the DCU. Define this macro |
| GFX_MGR02_ENABLE_LOG | Enable performance log capturing for gfx02 |

SOFTWARE target:

| Macro | Definitions |
| --- | --- |
| GFX_MGR02_SOFTWARE _FB_ADDRESS | Frame buffer memory information for the software rendering layers. |

Open GL target:

| Macro | Definitions |
| --- | --- |
| GFX_MGR02_TEXT_COLOR_CODE_PREFIX | Prefix value to be used to insert a draw colour command in string. |
| GFX_MGR02_TEXT_SPACE_CODE_PREFIX | Prefix value to specify the insertion of number of blank pixels. When the graphics manager sees this command, the following char after the command is considered as number of pixels to advance in X direction. |
| GFX_MGR02_GLES_VERSION | Specify the version of the OpenGL API to be used |
| GFX_MGR02_MAX_IMG_TEXTURES | specify the maximum number of textures to be allocated for image |
| GFX_MGR02_MAX_TEXT_TEXTURES | specify the maximum number of textures to be allocated for text |
| GFX_MGR02_MAX_CHAR_TEXTURES | specify the maximum number of textures to be allocated for char |

Fujitsu target:

| Macro | Definitions |
| --- | --- |
| GFX_MGR02_IRIS_SIG_UNIT_ENABLED | Enable Signature Unit Used for titan iris unit |
| GFX_MGR02_IRIS_PIXEL_POKING | Feature provided for display monitoring. Display monitoring can be done by providing data for specific pixel locations. The data given by application will override the existing HMI |

AMBER target:

| Macro | Definitions |
| --- | --- |
| GFX_MGR02_CUSTOM _GDC_SYSINIT_INITIALIZER | The Spansion's driver is configured the GFX PLL to 360MHz by default. We can provide our custom value depends on the PLL configured in the start.c file. #define GFX_MGR02_CUSTOM_GDC_SYSINIT_INITIALIZER { \ 0U, \ 360000000U \ } The first parameter says the safety driver applicable or not. 0-> No safety Driver. The second one is Graphics PLL frequency in terms of Hz. |
| GFX_MGR02_AMBER _USE_MML_PE_SYNC | The macro GFX_MGR02_AMBER_USE_MML_PE_SYNC can be used in order to avoid waiting for GPU to complete the current rendering and issuing of commands to GPU. |
| GFX_MGR02_AUTO _ALLOCATE_FRACTIONAL _LAYER | The macro GFX_MGR02_AUTO_ALLOCATE_ FRACTIONAL_LAYER can be used to provide control to the graphics manager to decide the fractional plane of the Keep Layer widget element. By enabling this macro the graphics manager will move the element to another plane if there is an overlap within the Keep Layer widget elements. This macro is in disabled by default. |
| The below 3 macros can be used for include any files related to 3D rendering, initialize the 3D stuffs and uninitialized the 3D stuffs. GFX_MGR02_3D_MDL_INCLUDE_H | Provide the header file name. |
| GFX_MGR02_3D_MDL_INITIALIZE | Provide the function call-back to initialize the 3D model if any. |
| GFX_MGR02_3D_MDL_UNINITIALIZE | Provide the function call-back to uninitialized the 3D model if any. |
| GFX_MGR02_USER_DEFINED_FT2_MM | Define the below macro in order to use this utility. The Spansion's utility and freetype2 libraries are modified to support this. |
| GFX_MGR02_AMBER_FT2_CACHE_ENABLED | In order to improve the freetype2 text render time in Amber platform the Freetype2 cache APIs are used. By enabling this, you need to configure some parameter for the cache manger. The Cache option is enable by defining macro GFX_MGR02_AMBER_FT2_CACHE_ENABLED In addition to that configure some of the below parameters for cache manager. |
| GFX_MGR02_IRIS_INSTR_BUFF_SIZE | Mention the size of the memory to be allocated for instruction buffer to queue and execute Iris instructions. The required instruction buffer size depends on the amount and complexity of the render instructions and which synchronization instructions are used. A minimum recommended size is 8 KB. This memory is allocated in the VRAM. This macro must be defined for Amber platform |

OPEN VG target:

| Macro | Definitions |
| --- | --- |
| GFX_MGR02_VG_CACHE_ENABLED | To enable the caching option for OpenVG. For more info refer hmi_gfx_mgr02.cfg file |
| GFX_MGR02_VG_CACHE_SIZE | Defines the maximum and/or default size of the pool. The pool is maintained per layer or context basis. The size of the pool is determined during code generation but the generated value is sealed to GFX_MGR02_VG_CACHE_SIZE. Larger the cache size the larger the memory that will be required. Configure it to zero if no pool is required and with 0. |
| GFX_MGR02_VG_RENDER_QUALITY | Choose the rendering (VG_RENDERING_QUALITY)/image quality(VG_IMAGE_QUALITY) 0 - QUALITY_NONANTIALIASED 1 - QUALITY_FASTER 2 - QUALITY_BETTER |
| GFX_MGR02_PRECREATE_FONTS | Generally the font glyphs are created, drawn and destroyed by graphics manager. But this will result in a performance hit because creation and destroy will be time consuming. So this configuration is to override the default configuration and allow the graphics manager to create all the glyphs during start up itself. This will improve the speed o maximum extend but the drawback is that more memory requirement and start-up time The other way is to cache a limited set of glyphs but it is not implemented currently. valid options are FONTS_NONE - No pre creation FONTS_BITMAP - pre-create only bitmap fonts (not supported) FONTS_SVG - pre-create only vector fonts FONTS_BITMAP¦FONTS_SVG - pre-create both vector and bitmap fonts |
| GFX_MGR02_OVG_ BUFFER_W_ALIGN_BYTES | Define the alignment required in bytes for OpenVG frame buffers. Currently this is used only for dynamic buffer allocation in Kepler DCU and is also defaulted to 8bytes or 32bits. So configure this only if a change is required. |
| GFX_MGR02_ ENABLE_FSL_EXTENTIONS | Enable for FreeScale Rainbow platform (MPC5645S). Additionally enable FSL_CONST_IMAGES to VG to load images directly from flash without creating in video RAM. |
| GFX_MGR02_RENESAS_OPENVG_LIB | Enable this if the target is Renesas SH7269 |
| GFX_MGR02_VG_MIRROR_OUTPUT | Enable mirroring of the output. Without this OpenVG layers will look upside down due to the origin difference between GDT and OpenVG. |
| GFX_MGR02_DAR_RECT_PROXIMITY | Proximity value for rectangles if a rectangle comes closer with in this proximity value; they get combined as a single dirty rectangle. This option is to reduce the number of dirty rectangles. valid ranges are from 0 to XX value of 0 means no proximity for dirty rectangles and multiple dirty rectangles gets combined only if they actually overlap |
| GFX_MGR02_DAR_ MAX_RECTANGLES | Threshold values to stop the dirty area redraw and do a full update instead. Because dirty area update on a large number of rectangles may be slower than doing a full update. Valid values are from 0 – XX Value 0 means option is disabled and dirty area update is performed regardless of number dirty rectangles. |
| GFX_MGR02_DEFAULT _BACKGROUND_COLOR | Define the 32-bit default background colour. Any area of the display that does not have an active pixel will be set to this colour, and it will be blended with other layers if those layers have selected the alpha Blending option. This is the value that will be set at cold/warm/wakeup Initialization. For more info refer hmi_gfx_mgr02.cfg file. |
| GFX_MGR02_ARABIC _LANG_SUPPORT_NEEDED | Enable the macro if bidi processing of strings has to be enabled for Arabic language support. |
| GFX_MGR02_INLINE_INIT | When using OpenVG/GL the init sequence will take a considerable time to complete. So when using DI-Kernel it may affect the task execution time boundary. So this macro provides an option to switch between an inline initialization GFX_MGR02_INLINE_INIT (1) or a delayed initialization in pre-emptable graphics task GFX_MGR02_INLINE_INIT (0) When using pre-emptive OS such as Linux, this macro can be set to GFX_MGR02_INLINE_INIT (1) always. If this macro is not defined, its default behaviour is 1. For OpenVG/GL - GFX_MGR02_INLINE_INIT (0) 2. For other targets - GFX_MGR02_INLINE_INIT (1) |
| GFX02_DEBUG_INFO | Enable to print dcu/2d-ace info prints |
| GFX02_DEBUG_ERROR | Enable to print dcu/2d-ace error prints |
| GFX02_DEBUG_CONTEXT | Enable to print context module prints |
| GFX02_DEBUG_OVG | Enable to print OpenVG module prints |
| GFX02_DEBUG_DAR | Enable to print dirty area module prints |
| GFX_MGR02_DIRTY _REGION_REDRAW_DEBUG | Enable to draw dirty area rectangles on top of the frame buffer for debugging |
| GFX_MGR02_SUPPORT_OPENVG_BMPFONT_FOR_SCALING | Define this macro if text to be rendered in graphics task context instead of eDMA interrupt handler. Used only for DCU platform. Should be defined if external font engine is used for text rendering. Also, define it if OpenVG text scaling is required (GFX_MGR02_SUPPORT_OPENVG_BMPFONT_FOR_SCALING). |
| GFX_MGR02_THEME_ENABLED | Different themes can be added using GDT tools. Based on the active theme set, images in that theme will be displayed. To enable this feature, below macro has to be defined. Refer hmi_gfx_mgr02_theme.c for interface. Currently support added for OpenVG platform. |
| GFX_MGR_APPEND_BASEDON_APP_IP | the text needs to be clipped from Right/Left based on the value returned from application |

Generic:

| Macro | Definitions |
| --- | --- |
| GFX_MGR02_TOUCH_EVENT_Q_SIZE | Specify the size of the touch event queue. When touch driver notifies touch events to graphics manager, it gets stored inside a queue for later process by graphics manager on next call to function hmi_gfx_mgr02_layout_build_screen. |
| GFX_MGR02_CREATE_CS GFX_MGR02_INIT_CS GFX_MGR02_ENTER_CS GFX_MGR02_LEAVE_CS GFX_MGR02_DELETE_CS | Since the touch events are put in to a queue and later processed by a different task, there is a possibility that the caller of the hmi_gfx_mgr02_layout_push_touch_event() and process function hmi_gfx_mgr02_layout_build_screen() might be running in a two different priority tasks which can get pre-empted. In such case above definitions allow the creation of a semaphore, mutex etc. to provide a locking mechanism to the touch event queue. Below sample implementation shows creation of a critical section for windows. Similar implementation could be followed for other RTOS |
| GFX_MGR02_IF_APP_PRESENTATION_RATE | The rate in milli seconds at which the presentation task to be scheduled. |
| GFX_MGR02_IF_TASK_RATE | The rate at which the hmi_gfx_mgr02_if_KSTimedTask() is scheduled by the application/RTOS more info refer .cfg file |
| GFX_MGR02_BMPFONT | Default with internal bitmap font |
| GFX_MGR02_DEFAULT_CHAR | set a default character code to replace a missing character |
| GFX_MGR02_DELEM_INVALID_INDEX_ENABLED | To set all the dynamic image and widget init index to invalid values. If not defined they will be initialised to current selection in GDT |
| GFX_MGR02_EXT_FONT_ENGINE | Specify the font engine as any one of below. GFX_MGR02_BMPFONT GFX_MGR02_SPANSION_FT2 GFX_MGR02_FREETYPE GFX_MGR02_DIGITYPE GFX_MGR02_MONOTYPE |
| GFX_MGR02_ENABLE_HW_DITHERING | Enable this to support hardware dithering implemented for Renesas, Yukawa 2.0 and Spansion Amber targets |
| GFX_MGR02_DIRTY_REGION_REDRAW | Defining this will enable dirty area redraw. Uses additional RAM to store the elements previous status and next status etc. for calculations |
| GFX_MGR02_THAI _LANG_SUPPORT_NEEDED | Enable the macro if Thai language Support is needed. GSUB table in font is used for Thai text rendering. |
| GFX_MGR02_CPU_ARABIC_JOINING_PATCH | In Kepler platform when text rendering mechanism used "GFX_MGR02_DMA_CPU_RENDER" and found 1 pixel gap between the Arabic characters then enable below macro GFX_MGR02_CPU_ARABIC_JOINING_PATCH which moves every joining characters and extra pixel to the left to avoid one pixel gaps observed in some fonts |

## Configuring hmi_gfx_mgr02_clut_tables.cfg file

| Macro | Definitions |
| --- | --- |
| GFX_MGR02_PROD_SPEC_CLUT_TABLE_NUM | The size is added to the total image/ text clut size. If the total clut size is greater than the available CLUT RAM space all cluts will be loaded dynamically based on the current image, widget, text... |
| GFX_MGR02_PROD_SPEC_CLUT_TABLE | Configure this macro as per project requirements. Example: #define GFX_MGR02_PROD_SPEC_CLUT_TABLE_NUM (2) #define GFX_MGR02_PROD_SPEC_CLUT_TABLE { &fuel_gradiant_clut1[0], 57 , 0 } ,{ &fuel_gradiant_clut2[0], 57 , 0 } |
| GFX_MGR02_NEED_CLUT_CREATION_FOR_NORMAL_FILLSCREEN | Set this macro, if regular fill screen elements need CLUT creation. |

## Configuring hmi_gfx_mgr02_fonts.cfg file

| Macro | Definitions |
| --- | --- |
| GFX_DEFAULT_FONT_DATA_PREFIX | Configure this macro as per project requirements. Example: #define GFX_HELVETICANEUELT35_DATA_PREFIX __declspec(section "QSPI0_FLASH") #define GFX_HELVETICANEUELT23_DATA_PREFIX __declspec(section "QSPI0_FLASH") |
| GFX_MGR02_FONT_TABLE_PREFIX | Configure this macro as per project requirements. |
| GFX_FONT_DATA_PREFIX | Configure this macro as per project requirements. |

## Configuring hmi_gfx_mgr02_safety.cfg file

| Macro | Definitions |
| --- | --- |
| GFX_MGR02_SIG_UNIT_ THRESHOLD_ERR_CNT | If signature error count reaches the GFX_MGR02_SIG_UNIT_THRESHOLD_ERR_CNT, display blank occurs |
| GFX_MGR02_SIG_UNIT _THRESHOLD_ERR_RST_CNT | No of correct frames which will reset the signature error count to zero, if configured as 0, then no reset of error count happens |

## Configuring hmi_gfx_mgr02_warp_tables.cfg file

Please refer the file hmi_gfx_mgr02_warp_tables.cfg file for details.

## Entry Points

NA

## Application Interfaces

| Function | Description |
| --- | --- |
| hmi_gfx_mgr02_layout_freeze_touch_event | Make the active touch for passed screen (if any) invalid. All further events are marked with GFX_MGR02_MOUSE_OUTOF_BOUND flag. Please make sure that the critical section/mutex configurations are done correctly in hmi_gfx_mgr02.cfg for proper operation. Please refer hmi_gfx_mgr02.$cfg for sample configuration for windows & Linux (llok for config items GFX_MGR02_CREATE_CS, GFX_MGR02_INIT_CS, GFX_MGR02_ENTER_CS, GFX_MGR02_LEAVE_CS, and GFX_MGR02_DELETE_CS). |
| Graphics Get |  |
| hmi_gfx_mgr02_layout_get_char_metrics | Gets character metrics |
| hmi_gfx_mgr02_layout_get_obj_area | Gets element area |
| hmi_gfx_mgr02_layout_get_obj_dim | Gets element dimension (width & height) |
| hmi_gfx_mgr02_layout_get_obj_pos | Gets element position |
| hmi_gfx_mgr02_layout_get_def_obj_transformation | Gets the elements default transformation matrix at design time |
| hmi_gfx_mgr02_layout_get_obj_transformation | Gets the elements current transformation matrix |
| Tasks |  |
| Graphics_Task | Rendering Task |
| Graphics_Task_Done | Rendering Task competed callback |
| hmi_gfx_mgr02_if_KSTimedTask | Presentation tick rate |
| hmi_gfx_mgr02_if_start | Starts the graphics manager |
| hmi_gfx_mgr02_if_shutdown | Request a shutdown |
| hmi_gfx_mgr02_if_is_ready_to_sleep | Inspect the graphics manager is shutdown |
| Callbacks (implemented by user & called by gfx02) |  |
| hmi_gfx_mgr02_load_user_layer_def_info | Called by graphics manager if it encounters a USER_TYPE image element. Within this function user is expected to load the image details Width, Height, BPP and pointer to pixel data. Function shall return TRUE if the image has changes since last call. |
| hmi_gfx_mgr02_app_touch_event_handler | Called by Graphics manager if a touch falls inside a touch enabled widgets. The element ID, touch event and co-ordinate information is passed |

## Target System Requirements

### IGDT design and code generation

Windows 7 operating system

Latest .Net runtime software (Can be downloaded from below URLs)

https://www.microsoft.com/en-in/download/details.aspx?id=29

https://www.microsoft.com/en-in/download/details.aspx?id=5582

### Target build tools

This package shall be integrated as a part of application software, so it needs not any build tools separately.

## Metrics

NA

## Integration Checklist

The following checklist is to be used by the package user to help ensure correctness of GDT design and configuration.

### GDT Project Settings

General

| S.No | Checklist Item | Yes/No/NA |
| --- | --- | --- |
| 1 | Ensure CodeGenOptions1-> Wide Char is enabled, if project contains non English languages. |  |
| 2 | Ensure CodeGenOptions1 -> DCU Multicolor Text is enabled, if any widget contains multi colors (different format of child elements). |  |
| 3 | Ensure CodeGenOptions1->Dyn elm idx gen is enabled to generate indices of dynamic widgets/images as enum values. |  |
| 4 | Ensure CodeGenOptions1->Use File System is enabled, if the target uses external memory with file system. (If enabled, IGDT generates files under directory Code Generation Path\gresfiles\*) |  |
| 5 | Ensure CodeGenOptions1->Little Endian is enabled to generate data in Little Endian format. (If not enabled, data will be generated in Big Endian format. Please check the platform matrix for details). |  |
| 6 | Ensure CodeGenOptions1->Gen RGBA is enabled to generate data in RGBA format. (Default is ARGB format) |  |
| 7 | Ensure CodeGenOptions1->GSUB table generation is enabled, if project contains THAI language |  |
| 8 | Ensure CodeGenOptions1->GPOS table generation is enabled, if project contains Arabic language. |  |
| 9 | Ensure CodeGenOptions1->DCU Text Render is set to Pre-render N DMA, if project contains Arabic language and using only static string. (or) Ensure CodeGenOptions1->DCU Text Render is set to CPU N DMA, if project contains Arabic language and using only dynamic string. (or) Ensure CodeGenOptions1->DCU Text Render is set to None, if project contains no Arabic language. |  |
| 10 | Ensure language translation sheet is configured in Language Options->Language Path, if CodeGenOptions1->DCU Text Render is set to Pre-render N DMA. |  |
| 11 | Ensure CodeGenOptions1->No of Pages is set as per requirements |  |
| 12 | Ensure CodeGenOptions1->Page Size(Bytes) is set as per requirements |  |
| 13 | Ensure CodeGenOptions1->LSH Version is set to Version 1 (or) Ensure CodeGenOptions1->LSH Version is set to Version 2 |  |
| 14 | Ensure CodeGenOptions1->Rotation Type is set to None, if no rotation feature required. (or) Ensure CodeGenOptions1->Rotation Type is set to Clockwise90Deg, if rotation feature required in clock direction. (or) Ensure CodeGenOptions1->Rotation Type is set to AntiClockwise90Deg, if rotation feature required in anti-clock direction. |  |
| 15 | Ensure CodeGenOptions1->RLE Threshold (%) is set as per requirements |  |
| 16 | Ensure CodeGenOptions1->Char Search Mode is set to LINEAR SEARCH, if font char ranges are continuous. (or) Ensure CodeGenOptions1->Char Search Mode is set to BINARY SEARCH, if font char ranges are discrete. (or) Ensure CodeGenOptions1->Char Search Mode is set to HYBRID SEARCH, if font char ranges are mixed (few ranges are continuous and others not). |  |
| 17 | Ensure Language Options->Language Path is set to a valid language input file (XLS), if project need to generate language data from GDT. (Template file available in \ui.core.iceapple.doc\language.core.01\design\ language_transtable_template.xls) |  |
| 18 | Ensure Language Options->Identifier Prefix is set as per project requirements. |  |
| 19 | Ensure Language Options->Maximum File Count is set as per project requirements. |  |
| 20 | Ensure Language Options->Maximum File Size (Bytes) is set as per project requirements. |  |
| 21 | Ensure Language Options->Use Internal Buffer is set as per project requirements. |  |
| 22 | Ensure Language Options->Readable DB file is set as per project requirements. |  |
| 23 | Ensure Language Options->Optimized DB is set as per project requirements. |  |
| 24 | Ensure Language Options->Language Driver Use is set as per project requirements. |  |
| 25 | Ensure Language Options->Create Language Segment Data is enabled, if project need to create language segment data in binary file. (This feature is under evaluation and advised to not use) |  |
| 26 | Ensure GUI Skinning->Image Themes->Theme Folder in Target is set to a valid directory name, where all the generated image data files will be located in target. (Currently Multi theme feature supported in OpenVG platform only) |  |
| 27 | Ensure GUI Skinning->Image Themes->Theme Folder in PC is set to a valid directory in local PC which contains all theme related image files. (Currently Multi theme feature supported in OpenVG platform only) |  |
| 28 | Ensure GUI Skinning->Image Themes->Default Theme is set to a valid directory, if project uses Multi Theme feature. (Currently Multi theme feature supported in OpenVG platform only) |  |
| 29 | Ensure GUI Skinning->Layout Themes are configured, if project uses Multi Layout feature. (Currently Multi layout feature supported in OpenVG platform only) |  |
| 30 | Ensure all the multi-line break characters are configured under Multiline tab. |  |
| 31 | Ensure all the windows fonts are installed before code generation. (Advised to install all the fonts used in projects, if font engine is set to Windows) |  |
| 32 | Click menu Report->Generate Layer Usage Report -> For All to generate RAM optimization report which contains details of GDT elements and its size in memory. |  |
| 33 | Ensure Mono Platform->Vertical Packing is configured, if project uses Mono platform |  |

Optimization

| 1 | Ensure CodeGenOptions1 -> Skip Font ROM Generation is enabled to skip generating font bitmap files. (Should actually be unchecked during integration generation to make sure the font data is generated) |  |
| --- | --- | --- |
| 2 | Ensure CodeGenOptions1 -> Font Rom Optimization is enabled to generate optimal size bitmaps for font characters. |  |
| 3 | Ensure CodeGenOptions1->RAM Optimization is enabled to decrease the RAM usage for dynamic text buffer allocation. |  |
| 4 | Ensure CodeGenOptions1->RAM Optimization->Skip Multiline Texts is enabled to skip multiline text elements while optimizing RAM. |  |
| 5 | Ensure CodeGenOptions1->Skip Image Optimization is enabled to skip generating optimal image data. (Optimal image data requires less memory) |  |
| 6 | Ensure Mono Platform->Quantize 4BPP Font as 2BPP is configured, if project uses Mono platform. |  |

Features vs. Platforms matrix

| # | GDT project settings / Platform | Dcu | DcuLite | OpenVG | Software | OpenGL | Fugitsu | Amber | Mono |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | CodeGenOptions1-> Skip Image Optimization | Y | Y | Y | Y | Y | Y | N | Y |
| 2 | CodeGenOptions1 -> Skip Font ROM Generation | Y | Y | Y | Y | Y | Y | Y | Y |
| 3 | CodeGenOptions1 -> Font Rom Optimization | Y | Y | Y | Y | Y | Y | Y | N |
| 4 | CodeGenOptions1-> RAM Optimization | Y | Y | Y | Y | Y | Y | Y | Y |
| 5 | CodeGenOptions1-> RAM Optimization->Skip Multiline Texts | N | N | Y | N | N | N | N | N |
| 6 | CodeGenOptions1-> Wide Char | Y | Y | Y | Y | Y | Y | Y | Y |
| 7 | CodeGenOptions1 -> DCU Multicolor Text | Y | N | N | N | N | N | N | N |
| 8 | CodeGenOptions1->Dyn elm idx gen | Y | Y | Y | Y | Y | Y | Y | N |
| 9 | CodeGenOptions1->Use File System | Y | Y | Y | Y | Y | Y | Y | N |
| 10 | CodeGenOptions1->Little Endian | Y | Y | Y | Y | Y | Y | Y | N |
| 11 | CodeGenOptions1->Gen RGBA | Y | Y | Y | Y | Y | Y | Y | N |
| 12 | CodeGenOptions1->GSUB table generation | N | N | N | N | N | N | Y | N |
| 13 | CodeGenOptions1->GPOS table generation | N | N | Y | N | N | N | N | N |
| 14 | CodeGenOptions1->DCU Text Render = CPU N DMA | Y | N | N | N | N | N | N | N |
| 15 | CodeGenOptions1->DCU Text Render = Pre-render N DMA | Y | N | N | N | N | N | N | N |
| 16 | CodeGenOptions1->DCU Text Render = None | Y | Y | Y | Y | Y | Y | Y | N |
| 17 | CodeGenOptions1->LSH Version = Version 1 & Version 2 | Y | Y | Y | Y | Y | Y | Y | Y |
| 18 | CodeGenOptions1->Rotation Type = None | Y | Y | Y | Y | Y | Y | Y | N |
| 19 | CodeGenOptions1->Rotation Type = Clockwise90Deg | Y | Y | N | Y | Y | N | N | N |
| 20 | CodeGenOptions1->Rotation Type = AntiClockwise90Deg | Y | Y | N | Y | Y | N | N | N |
| 21 | CodeGenOptions1->RLE Threshold (%) | Y | Y | Y | Y | Y | Y | N | Y |
| 22 | CodeGenOptions1->Char Search Modes | Y | Y | Y | Y | Y | Y | Y | Y |
| 23 | GUI Skinning->Image Themes->Theme Folder in Target | N | N | Y | N | N | N | N | N |
| 24 | GUI Skinning->Image Themes->Theme Folder in PC | N | N | Y | N | N | N | N | N |
| 25 | GUI Skinning->Image Themes->Default Theme | N | N | Y | N | N | N | N | N |
| 26 | GUI Skinning->Layout Themes | N | N | Y | N | N | N | N | N |
| 27 | Multiline tab settings | N | N | Y | N | N | N | N | N |
| 28 | Mono platform tab settings | N | N | N | N | N | N | N | Y |

### GDT Project Design

| S.No | Checklist Item | Yes/No/NA |
| --- | --- | --- |
| 1 | Screens: Have screen element is added to directly to any animation element? If yes have enabled "Animation" property |  |
| 2 | Screens: Have screen element is used for menu? If yes, have configured "EventHandler" and "PresentationHandler" as given below: EH --> hmi_menu_rx_button_handler PH --> hmi_menu_presentation_handler |  |
| 3 | Widgets: Have Widget element is added to directly to any animation element? If yes have enabled "Animation" property |  |
| 4 | Widgets: Have widget element is used for multicolor widget? If yes have maintained without overlapping between the elements Note: Specific to Kepler 1.0 (Spectrum/Rainbow)only. |  |
| 5 | Widgets: Does merge widget has if it has all indexed images? If yes, then it should have only one CLUT group Note: Specific to Kepler 1.0 (Spectrum/Rainbow)only. |  |
| 6 | Images: Does indexed images are used? If yes, make ensure images should have number of colors <= as per below table: 1BPP = 2 2BPP = 4 4BPP = 16 8BPP = 256 APAL16 = 256 |  |
| 7 | Fonts: Does configured "NumChars" property as per requirement? (observed some where it is configured as 224) |  |
| 8 | Fonts: Have default font feature used in font groups? If yes, have maintained common height between all child fonts? |  |
| 9 | Fonts: Have configured all the Unicode ranges required for the using languages? |  |

### Graphics Manager

### General (Features/Macros applicable for all platforms)

| S.No | Checklist Item | Yes/No/NA |
| --- | --- | --- |
| 1 | If scroll is required, enable "GFX_MGR02_HORIZONTAL_SCROLL_ENABLED" (make ensure that it should not be rotated display) |  |
| 2 | If clut entries are greater than 2048 colors then make ensure enable <GFX_MGR02_DYNAMIC_CLUT_LOAD_SUPPORT_NEEDED> since we have only 8KB of CLUT RAM |  |
| 3 | Update #define GFX_MGR02_GRAM_OVERAP_HANDLING (GFX_MGR02_BUILD_IN_VBLANK) as #define GFX_MGR02_GRAM_OVERAP_HANDLING (GFX_MGR02_BUILD_DCU_VBLANK) for non-overlapped GRAM layers will build in background. (for latest ui.core.gdt baselines) |  |
| 4 | If OpenVG rendering objects are used in design then it will be better to enable <GFX_MGR02_VG_CACHE_ENABLED> (to cache the most used objects) |  |
| 5 | Configure <GFX_MGR02_DEFAULT_CHAR> if runtimes any unsupported Unicode exist in a dynamic string |  |
| 6 | If "Pre render N DMA" text rendering mechanism used then do not enable the <GFX_MGR02_ARABIC_LANG_SUPPORT_NEEDED> since Arabic string is already processed by bidi algorithm while GDT code generation |  |
| 7 | If project uses font group feature with default font feature then enable <GFX_MGR02_DEFAULT_FONT> |  |
| 8 | <GFX_MGR02_GRAM_SIZE> size should not exceed the 1MB |  |
| 9 | Enable <GFX_MGR02_DELEM_INVALID_INDEX_ENABLED> if all the dynamic images and widgets are need to be set to invalid index by default |  |
| 10 | In general, right aligned texts are clipped from left if string gets width is greater than > avail width and for left/center aligned texts characters are clipped from right. But if application wants to change this at run time they can enable <GFX_MGR_APPEND_BASEDON_APP_IP> configure with respective callback function. |  |

### Kepler2 platform

| S.No | Checklist Item | Yes/No/NA |
| --- | --- | --- |
| 1 | Ensure macro GFX_MGR02_CUSTOM_GDC_SYSINIT_INITIALIZER is configured. The Display Controller clock is set through this macro to below structure variable. typedef struct { MM_U32 ResourceLock; /**< Bitfield that describes resources allocated by safety driver, Default: 0 */ MM_U32 GfxPll; /**< Frequency of GFX PLL (for Pixel Clock generation) in Hertz, Default: 400000000u*/ } MML_GDC_SYSINIT_INFO; Recommended to set this to {0u, 400000000u}, but can be configured based on macro used and project requirements. |  |
| 2 | Ensure macro GFX_MGR02_IRIS_INSTR_BUFF_SIZE is configured. (Mention the size of the memory to be allocated for instruction buffer to queue and execute Iris instructions. The required instruction buffer size depends on the amount and complexity of the render instructions and which synchronization instructions are used. A minimum recommended size is 8 KB. This memory is allocated in the VRAM. This macro must be defined) |  |
| 3 | Ensure macro GFX_MGR02_AMBER_USE_MML_PE_SYNC is configured, if want to avoid waiting for the GPU to complete the current rendering and issuing of commands to GPU. |  |
| 4 | Ensure below macros are configured to configure Draw Engine. GFX_MGR02_DE_AREA_WIDTH Set the Width of the Draw area based on your requirement. GFX_MGR02_DE_AREA_HEIGHT Set the Height of the Draw area based on your requirement. GFX_MGR02_DE_ALPHABUF_SIZE The Spansion's recommended size of the Alpha Buffer is calculated by GFX_MGR02_DE_ALPHABUF_SIZE = (Width x Height x Bit Depth x 2)/8, Whereas Bit Depth is equal to 1 or 2 or 4 or 8. GFX_MGR02_DE_PATHBUF_SIZE The Size of the path buffer must be power of 2. |  |
| 5 | Ensure below macros are configured, if used Freetype fonts. GFX_MGR02_USE_EXT_FONT_ENGINE Set this macro to render the font from external freetype engine. GFX_MGR02_EXT_FONT_ENGINE Set this macro to GFX_MGR02_SPANSION_FT2. Ensure the macro GFX_MGR02_USER_DEFINED_FT2_MM is enabled, if the heap for freetype2 memory allocated in VRAM. (Recommended to enable this macro, if font engine need more than 300KB heap). Ensure the macros are defined in start and end of VRAM memory section created for the Freetype2 engine. __ghsbegin_FT2HEAP __ghsend_FT2HEAP |  |
| 6 | Ensure GFX_MGR02_AMBER_FT2_CACHE_ENABLED is enabled, if want to improve the freetype2 texts render time. If enabled configure below macros as well. GFX_MGR02_AMBER_FT2_CACHE_MAX_FACES Maximum number of opened FT_Face objects managed by this cache instance. Use 0 for default value. GFX_MGR02_AMBER_FT2_CACHE_MAX_SIZES Maximum number of opened FT_Size objects managed by this cache instance. Use 0 for default value. GFX_MGR02_AMBER_FT2_CACHE_MAX_BYTES Maximum number of bytes to use for cached data nodes. Use 0 for default. |  |
| 7 | GFX_MGR02_SPANSION_EXTRAM_USAGE_CFG In order to allocate frame buffer for the merge widgets of user choice in Hyper RAM; Define the macro GFX_MGR02_SPANSION_EXTRAM_USAGE_CFG. Also provide the start/end address of Memory to be used for this in linker script. //__ghsbegin_gfx02_workbuff_start //__ghsend_gfx02_workbuff_end It is advised to use this option for small and less frequent data change widgets. |  |
| 8 | 3D(Brontes Engine) Settings The below macros can be used with files related to 3D rendering, initialize/uninitialize the 3D stuffs. GFX_MGR02_3D_MDL_INCLUDE_H Provide the header file name. GFX_MGR02_3D_MDL_INITIALIZE Provide the function call-back to initialize the 3D model if any. GFX_MGR02_3D_MDL_UNINITIALIZE Provide the function call back to uninitialize the 3D model if any. |  |
| 9 | Ensure GFX_MGR02_DIRTY_REGION_REDRAW is enabled to enable dirty area redraw. The Dirty area redraw feature in Kepler2 platform is allowed only for Clipped type merge widgets. This can be set in the GDT tool by making the Clip=TRUE for the merge widgets. |  |
| 10 | Ensure GFX_MGR02_ENABLE_HW_DITHERING is enabled, if need to support hardware dithering. |  |

## Package

The following checklist is to be used by the package user to help ensure correctness of package integration and operation.

| S.No | Checklist Item | Yes/No/NA |
| --- | --- | --- |
| 1 | Ensure the following configuration files are generated from its templates - hmi_gfx_mgr02.cfg - hmi_gfx_mgr02_clut_table.cfg - hmi_gfx_mgr02_font.cfg - hmi_gfx_mgr02_safety.cfg - hmi_gfx_mgr02_warp_tables.cfg - panel_definition.h |  |
| 2 | Ensure the below files are generated from IGDT. - hmi_gfx_mgr02_cfg.h - hmi_gfx_mgr02_clut_table.h - hmi_gfx_mgr02_clut_table.c - hmi_gfx_mgr02_config.dat - hmi_gfx_mgr02_font.c - hmi_gfx_mgr02_font.h - hmi_gfx_mgr02_image_data.h - hmi_gfx_mgr02_ovg_table.dat - hmi_gfx_mgr02_svg_table.dat - hmi_gfx_mgr02_vgfont.c - hmi_logic_state_handler.dat - hmi_logic_state_handler.inc |  |
| 3 | Ensure the image data files are generated from IGDT. code_gen_path\<Image_element_name.dat> |  |
| 4 | Ensure the image data files are generated in gresfiles directory, if Use File System option is selected in IGDT project settings. code_gen_path\gresfiles\<image_element_name.dat> |  |
| 5 | Ensure that the IGDT generated files are copied into the configured Theme Folder in Target directory in external memory, if Use File System option is selected. |  |
| 6 | Ensure that the font files (*.ttf) are copied into external memory, if font elements are configured with External Font Engine property enabled. |  |
| 7 | Ensure that the IGDT generated image files are copied in the same directory order, if layout/image theme features are enabled. |  |

## Appendix B – Recommended Practices

- Application engineers advised to use latest version of ui.core.gdt component from RTC.

- Refer previous project design and configuration with target platform.

For example, if the new project planned to use OpenVG platform, refer other projects/ previous projects which used OpenVG platform.

- Ensure to verify the checklists when required.

- While generating code, IGDT triggers a process gfx_cmdline_coder.exe to generate code. Ensure this task is not closed manually.

- Advised to enable “Skip Font ROM Generation”, if font configuration has no change since last code generation.

- Refer the code generation log file hmi_gfx_mgr02_code_gen.txt for details, if any warnings/errors.

## Appendix C – Revision History

| Revision Level | RTC Work item | Date | Description | Updated by (CDSID) |
| --- | --- | --- | --- | --- |
| 1.0 | 816080 | 01-Mar-17 | Initial version | kkubendi |
| 1.1 | 864039 | 17-Jul-17 | Updated for Mono platform | kkubendi |